Organization Logo

FI.145.9999

PART- 145

MAINTENANCE ORGANIZATION MANUAL

# MAINTENANCE ORGANIZATION EXPOSITION

Change 18 / 3.11.2025

CONTACT INFORMATION

ANONYMOUS LTD

Airport Road

01530 Vantaa

+358 555 6767

Responsible manager:

John the Leader

+358 555 1122

jj@a.fi

Technical manager

Anton Anonymous

+358 123,456

aa@a.fi

## TABLE OF CONTENTS

LIST OF CHANGES 	4

IN EFFECT FOR THE CURRENT PAGES LIST 	5

CONCEPTS AND ABBREVIATIONS 	5

PART 1: MANAGEMENT 	6

  - Operation permission validity 	6

  - Responsible leader given by insurance 	6

  - Safety- and quality policy 	7

  - Executive staff 	7

    - Responsible leader 	7

    - Admitted person in charge 	7

    - About quality equivalent person 	7

    - Substitution arrangements 	7

  - Executive personnel functions and responsibilities 	8

    - Responsible leader 	8

    - Technical leader 	8

    - Quality Manager 	8

  - Organization chart 	9

  - Maintenance certificate givers and support staff 	9

  - In hand existent workforce 	9

  - Overview of premises 	10

    - Workshop maintenance facilities 	10

    - Permanent line maintenance facilities 	10

    - Temporary line maintenance locations 	10

    - Line maintenance location seal of approval and use 	10

  - Organization maintenance capabilities and work scope 	12

    - Organization maintenance capabilities and their determination 	12

    - Organization work scope 	13

  - Changes notification 	14

  - Organization manual amendment procedure 	15

PART 2: MAINTENANCE METHODS 	16

  - Supplier and subcontractors judgment and control method 	16

  - Aircraft parts, equipment and materials inspection and approval 	16

  - Aircraft parts, equipment and materials management 	17

    - Rating 	17

    - Marking 	17

    - Storage 	17

    - Delivery maintenance operations 	18

    - Scrapping ( mutilation ) 	18

  - Tools and equipment acceptance 	19

    - General 	19

    - Substitute tool/equipment approval 	19

    - Tools/equipment manufacturing and approval 	19

  - Tools and equipment maintenance and calibration 	19

    - Tools and instruments registration and marking 	19

    - Calibrated measuring instrument id supervision 	20

    - Measuring instruments seal of approval to use 	20

  - Tools and equipment use (incl. alternative tools) 	21

  - Maintenance facilities purity rating 	21

  - Maintenance instructions, their for in step with holding and usability mixed contact TC holders instructions 	22

  - Repair methods 	23

    - Defects repairs 	23

    - Damage repairs 	23

  - Aircraft maintenance program compliance 	24

  - Mandatory airworthiness regulations concerning procedures 	24

  - Volunteers renovations concerning procedures 	24

  - maintenance design, Working documents and work steps 	25

    - Maintenance working documents 	25

    - Working documents filling 	25

    - Maintenance design 	25

    - Maintenance work stages and entries 	26

    - Approved maintenance points outside task random work 	26

  - Technical accounting supervision 	27

  - MAINTENANCE IN CONNECTION WITH DEFECTS DETECTED REPAIR OR TRANSFER 	27

  - Procedures maintenance certificates to give 	28

    - Maintenance certificate administration for aircraft 	28

    - Maintenance certificate administration for parts, for devices or for equipment 	29

    - In working order detached assemblies and equipment 	29

  - For the operator given maintenance documents 	29

  - Airworthiness as an obstacle those who are defects notification 	30

  - Defective aircraft parts restoration to stock 	31

  - Defective parts restoration for outsiders for contract partners 	31

  - Electronic maintenance accounting supervision 	31

  - Labor resources and maintenance plan comparison 	31

  - Critical maintenance tasks supervision 	31

  - Special maintenance and manufacturing methods 	33

  - Maintenance errors observation and repair methods 	33

  - Shift/task exchange methods 	34

  - Service information incorrectness or inaccuracies notification 	3 4

  - Production design methods 	34

PART L2: LINE MAINTENANCE CONCERNING ADDITIONAL METHODS 	35

L2.1 Aircraft parts, equipment, tools, equipment etc. supervision 	35

L2.2 For refueling, for de-icing etc. related methods in line maintenance 	35

L2.3 Defects and recurring defects tracking in line maintenance 	35

L2.4 Technical travel diary filling method in line maintenance 	35

L2.5 Pooled contracts and borrowed parts concerning methods 	35

L2.6 From the aircraft detached​ defective parts/equipment supervision 	35

L2.7 Critical tasks control and errors repair methods 	35

PART 3: QUALITY METHODS 	36

  - Quality system 	36

    - Quality assurance 	36

    - Schedules 	36

    - Reporting procedures 	37

    - Management tracking- and feedback system 	37

  - Methods and aircraft quality audits 	38

  - Audits and deviations correct measures 	38

    - Auditing 	38

    - Corrective measures 	39

    - NAA announced observations processing 	40

  - Maintenance certificate givers and support staff suitable for pain and training methods 	40

    - Qualification methods 	40

    - Training methods 	42

    - Type training internship organizing 	43

  - Personal register 	44

  - Quality reviewers 	44

  - Inspectors qualification 	44

  - PERSONNEL QUALIFICATION 	45

  - Aircraft and equipment maintenance exceptional procedures management 	45

  - Organization procedures deviation management 	46

  - SPECIAL PROFESSIONAL QUALIFICATIONS 	46

  - Outsiders maintenance teams supervision 	46

  - Human factors - training methods 	46

  - Personnel qualifications assessment 	47

PART 4: MAINTENANCE AGREEMENTS 	48

  - Service contract operators 	48

  - OPS operator regarding to be followed methods and paperwork 	48

  - OPS operator market documents filling 	48

PART 5: APPENDICES AND LISTS 	49- 50

# CHANGE LIST

| Change | Date | Changed pages and things | Approval reference (NAA Bottom or Technical Director ) |
| --- | --- | --- | --- |
| 0 | 01.09.09 | New handbook, new pages 1 - 49. |  |
| 1 | 10.03.10 | New pages: 1, 4, 5, 9, 13, 15, 28, 30, 43, 45 and list of attachments: -updated Commission setting (EUROPEAN UNION) 127/2010 MOE methods. -changed e.g. points HTAs , scope of work, implicit approval. |  |
| 2 | 09.08.10 | New pages: 1, 4-5, 9, 13 and annex 1. Added Cessna 525/525A scope of work. Made HTA- and human resource information update. |  |
| 3 | 27.05.11 | New pages: 1-6, 8-9, 11-12, 14-16, 23-25, 28-29, 31, 38, 41- 43, 47 and list of appendices. Human resource information (page 9) and HTAs and line maintenance locations have been updated (appendix 1). Indirect approval procedure. Made IH  NAA update. |  |
| 4 | 10.10.11 | New pages 1-6, 10, 15-19, 39, 42 and 43. Clarifications of method descriptions. |  |
| 5 | 01.09.12 | New pages 1-5, 7-11, 13-15, 17-18, 20, 23, 26-27, 33, 37-39, 43- 49 and annex 1. Methods clarifications ( ref . EASA and (NAA) |  |
| 6 | 15.05.14 | New pages 1, 4-5, 7, 9, 11-13, 14, 16, 17 19-20, 22, 24, 26, 30, 31, 35, 43-44, 46-49 and annex. Requirements updates European Union 593/2012. Clarifications of methods and updates to notes. |  |
| 7 | 01.09.15 | New pages 1,4,5,10 and 13. Line maintenance points, Vantaa oil road and Turku deleted likewise C14 Wipline deleted scope of work . |  |
| 8 | 27.06.16 | New pages 1, 4-5, 7-10, 13, 15, 17-18, 29-31, 33-35, 4, 46- 49. Updated ANNEX 1. Requirements updates European Union 1321/2014, EU 376/2014 and EU 2015/1536. Methodological clarifications and updates to the notes. |  |
| 9 | 19.08.16 | New pages 1, 4-5, 13, 29. European Union 1321/2014 increase For the issuer of the C maintenance certificate. |  |
| 10 | 10.02.17 | New pages 1, 4-5, 10, 13. Head office moved to Helsinki- Vantaa, Helsinki-Malmi changed as a branch office. Cessna 525/525A scope of work changed to repair level for maintenance. |  |
| 11 | 10.01.18 | New pages 1,4,5,7 Updated ANNEX 1 paragraph 5.5 Auditor changed and HTA personnel added |  |
| 12 | 24.05.18 | New pages 1-5,9,11,17,27,30,31,32,34,37,38,43,44,46- 49. Support staff -update, mixed voluntary reporting method. |  |
| 13 | 30.10.18 | New pages 1,4,5,6,7 Responsible Director changed |  |
| 14 | 30.09.19 | New pages 1,4,5,6 and 7 Responsible Manager mixed CQM changed |  |
| 15 | 19.01.21 | All pages, CL604 propagation scope of work, HTA list update |  |
| 16 | 15.04.21 | All pages. Organization name changed. Removed C172 and C180 scope of work. Added scope of work C14 Landing gear. |  |
| 17 | 01.08.21 | All pages. Deleted Helsinki-Malmi branch office and removed C5 and C7 from the scope of work. HTA list updated. | Traficom/9999999/ 05.00.18.01/2025 |
| 18 | 17.01.22 | All pages. Scope of work added sideways 13. CL-600-2B16 12-month maintenance packages . |  |

# IN EFFECT FOR THE CURRENT PAGES CATALOG

| Page | Rev.​ | Date | Page | Rev.​ | Date | Page | Rev.​ | Date | Page | Rev.​ | Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 18 | 3.11.2025 | 21 | 18 | 3.11.2025 | 41 | 18 | 3.11.2025 |  |  |  |
| 2 | 18 | 3.11.2025 | 22 | 18 | 3.11.2025 | 42 | 18 | 3.11.2025 |  |  |  |
| 3 | 18 | 3.11.2025 | 23 | 18 | 3.11.2025 | 43 | 18 | 3.11.2025 |  |  |  |
| 4 | 18 | 3.11.2025 | 24 | 18 | 3.11.2025 | 44 | 18 | 3.11.2025 |  |  |  |
| 5 | 18 | 3.11.2025 | 25 | 18 | 3.11.2025 | 45 | 18 | 3.11.2025 |  |  |  |
| 6 | 18 | 3.11.2025 | 26 | 18 | 3.11.2025 | 46 | 18 | 3.11.2025 |  |  |  |
| 7 | 18 | 3.11.2025 | 27 | 18 | 3.11.2025 | 47 | 18 | 3.11.2025 |  |  |  |
| 8 | 18 | 3.11.2025 | 28 | 18 | 3.11.2025 | 48 | 18 | 3.11.2025 |  |  |  |
| 9 | 18 | 3.11.2025 | 29 | 18 | 3.11.2025 | 49 | 18 | 3.11.2025 |  |  |  |
| 10 | 18 | 3.11.2025 | 30 | 18 | 3.11.2025 | *(50) | 18 | 3.11.2025 |  |  |  |
| 11 | 18 | 3.11.2025 | 31 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 12 | 18 | 3.11.2025 | 32 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 13 | 18 | 3.11.2025 | 33 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 14 | 18 | 3.11.2025 | 34 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 15 | 18 | 3.11.2025 | 35 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 16 | 18 | 3.11.2025 | 36 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 17 | 18 | 3.11.2025 | 37 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 18 | 18 | 3.11.2025 | 38 | 18 | 3.11.2025 |  |  |  | *The information is at the end of the manual attachments and from the lists (Appendix 1: Rev. 17 / 01.08.2021) |  |  |
| 19 | 18 | 3.11.2025 | 39 | 18 | 3.11.2025 |  |  |  |  |  |  |
| 20 | 18 | 3.11.2025 | 40 | 18 | 3.11.2025 |  |  |  |  |  |  |
| This manual has been reviewed, where applicable, to comply with EU Regulation No. 1321/2014, Annex II / Part-145 (26.11.2014) and its amendments. The manual is approved for use by the organisation when it has been approved in accordance with the MOE 1.11 procedure and the approval has been recorded in the change history. In Vantaa November 3, 2025, admitted responsible person Anton Anonymous |  |  |  |  |  |  |  |  |  |  |  |

## CONCEPTS AND ABBREVIATIONS:

| AD | Airworthiness Directive / Airworthiness Directive |
| --- | --- |
| ECI | EASA Emergency Conformity Information – publication |
| SIB | EASA Safety Information Bulletin - publication |
| NAA | Finland aviation safety authority / National Aviation Authority (NAA) = Traffic there is so-called competent authoritative or Competent Authority . |

# PART 1: DERIVATION

## LICENSE VALIDITY

Maintenance organization takes care of, that following license conditions (145.A.90) filled continually:

- we act continually Part-145 requirements by,

- EASA and NAA inspectors there is always the right to enter to inspect organization and offices,

- license there is no surrendered or off (cancelled permit will be returned for NAA ),

- NAA announced observations (145.A.95c) handling will be done Marten satisfactory way (more details in section 3.3.3).

## RESPONSIBLE LEADER'S GIVEN INSURANCE

VAKUUTUS

Tämä käsikirja viiteasiakirjoineen määrittelee Part-145.A.70:n mukaisen organisaation ja menetelmät, joihin NAA:n myöntämä Part-145 hyväksyntä perustuu. Allekirjoittanut on hyväksynyt nämä menetelmät ja soveltuvilta osilta niitä noudatetaan suoritettaessa Part-145 hyväksynnän ehtojen alaisia töitä tai tilauksia.

Hyväksyn, etteivät nämä menetelmät poista tarvetta noudattaa NAA:n ajoittain julkaisemia uusia tai muutettuja määräyksiä, mikäli nämä uudet tai muutetut määräykset ovat ristiriidassa näiden menetelmien kanssa.

Ymmärrän, että NAA hyväksyy organisaation varmistuttuaan siitä, että menetelmiä noudatetaan ja työn laatua ylläpidetään. Ymmärrän myös, että NAA varaa itselleen oikeuden kieltää organisaation Part-145-toimiluvan mukaisen toiminnan väliaikaisesti, rajoittaa Part-145 toimilupaa tai peruuttaa sen kokonaan, mikäli sillä on näyttöä siitä, ettei menetelmiä noudateta tai työn laatua ylläpidetä.

Vastuullinen johtaja

Johannes Johtaja

## SAFETY- AND QUALITY POLICY

SITOUMUS

Organisaatio sitoutuu ylläpitämään ja kehittämään lentoturvallisuutta, toiminnan tehokkuutta ja tasalaatuisuutta tässä käsikirjassa määriteltyjen menetelmien ja menettelytapojen mukaisesti. Kaikessa toiminnassa otetaan huomioon inhimillisiin tekijöihin liittyvät periaatteet ja rajoitteet sekä turvallisen toiminnan ensisijaisuus.

Huolto-organisaation henkilöstö sitoutuu noudattamaan hyväksyttyjä menetelmiä ja määräyksiä sekä laatu- ja turvallisuusnormeja. Lisäksi organisaation vastuullinen johto ylläpitää toiminta- politiikkaa, jossa organisaation jokaisella henkilöllä on oikeus ja velvollisuus puuttua havaitsemiinsa ongelmiin, virheisiin ja vaaratilanteisiin sekä tehdä avointa yhteistyötä laatuauditoijien ja ilmailuviranomaisten kanssa.

## EXECUTIVE STAFF

#### Responsible manager

Anonymous Ltd. there is appointed John the Leader Part-145.A.30a to suit responsible as a leader.

#### Admitted responsible person

Anonyymi Oy has appointed Anton Anonyymi as the responsible person in accordance with Part-145.A.30b. The job title for the position is Technical Director.

#### About quality equivalent person

Anonyymi Oy has appointed Mikko Mallikkaa as the person responsible for quality in accordance with Part-145.A.30c. The job title of the position is Quality Manager. He is also responsible for the organization's Compliance Monitoring Manager positions.

The Quality Manager is supported by contract auditor Tero Terävä, who conducts independent audits in accordance with Part-145.A.65c1.

#### Substitution arrangements

The Technical Director serves as the Deputy Director. The Technical Director is HTA Tero Terävä. The Quality Manager is replaced by the Responsible Director. During the replacement, only individual positions are transferred. functions, total liability remains permanent position with a nurse. Substitution duration Oh be no more than six weeks at a time.

## EXECUTIVE PERSONNEL FUNCTIONS AND RESPONSIBILITIES

#### Responsible manager

      - answer organization from the activity and licenses,

      - answer, that maintenance can be finance and make norms in accordance with,

      - answer organization safety- and quality policy,

      - answer NAA notification observational deviations processing and measures,

      - answer last on everyone quality deviations processing and corrective working about doing,

      - answer activity requiring about communication to aviation authorities.

#### Technical manager

      - answer workshop-, line- and equipment maintenance as a whole (incl. contracts),

      - answer, that maintenance will be done approved norms and methods in accordance with,

      - answer organization scope of work status and -capabilities maintenance,

      - answer personnel about education, competence and sufficiency to the task to work,

      - answer educational planning and - monitoring,

      - answer type training internship organizing (3.15),

      - supervises interns use,

      - answer business premises sufficiency, condition and suitability mixed for the use of approval,

      - answer maintenance-, measure- and tools sufficiency, condition and supervision,

      - answer materials acquisition-, handling- and extradition procedures correctness,

      - accepts organization own work instructions, maintenance instructions/lists and forms,

      - answer maintenance literature and instructions availability and for in step with feeling,

      - answer AD- and SIB publications mixed aviation regulations for the organization giving obligations,

      - answer aircraft and equipment technical accounting and files maintenance,

      - answer From MOE (with reference documents) mixed its maintenance, about changing and distribution,

      - answer exemptions about the acquisition and monitoring,

      - answer safety observations processing and further measures,

      - answer airworthiness endangering defects evaluation and notification,

      - participates quality audits and answer quality deviations corrective working about doing,

      - keep responsible leader for in step with organization from the activity and tasks,

      - takes care of tasks according to about communication to aviation authorities and to customers.

#### Quality Manager

      - answer requirements conformity supervision,

      - answer organization quality system and its feedback system supervision,

      - answer quality plan drafting and implementation (incl. quality training),

      - answer quality checks and audits organizing,

      - answer quality meetings organizing (3.1.2c),

      - answer corrective working supervision and if necessary their doings from rushing,

      - keep organization responsible persons consciously quality management obligations and deadlines,

      - answer personnel qualifications about assessments (3.14),

      - grant personnel qualifications and - authorizations,

      - answer personnel qualification and educational documents mixed from the personal register,

      - keep contact to aviation authorities to my own industry related in general in matters,

      - answer quality management documents from the presentation and recording.

## ORGANIZATIONAL CHART

Anonyymi Oy

Hallitus

Part-145 -organisaatio

VASTUULLINEN JOHTAJA

LAATUPÄÄLLIKKÖ

SOPIMUSAUDITOIJAT

TEKNILLINEN JOHTAJA

KORJAAMO-, LINJA- JA LAITEHUOLTO

HUOLTOTODISTEENANTAJAT MEKAANIKOT JA ASENTAJAT

## MAINTENANCE CERTIFICATE DONORS AND SUPPORT PERSONNEL

Certifying service providers staff ) and support staff ( support staff ) are presented in section 5.5. Their education, work experience and Qualification requirements are presented in section 3.4.

## IN HAND EXISTENT MANPOWER

| Task | Number | Part-145 booking h / year | Group other tasks B / year | Note |
| --- | --- | --- | --- | --- |
| Technical manager . + HTA | 1 | 800 B | 800 B | - |
| HTA and maintenance engineer | 8 | 6400 B | 1600 hrs | - |
| Maintenance personnel total : | 9 | Total of. 7200 B | - | - |
|  |  |  | - |  |
| Responsible manager ( take ) | 1 | 100 | Main activity |  |
| Quality Manager ( take ) | 1 | 300 | Main activity |  |
| Auditor ( take ) | 1 | 50 - 100 | 50 - 100 |  |

HTAs work if necessary also Part-145.A.30(h)(1i) in accordance with as support persons demanding in motor-powered aircraft repair shops (1.9.2, section 1 A2).

Labor operational planning is presented in the 2.22.

## GENERAL DESCRIPTION ABOUT THE OFFICES

#### Workshop maintenance facilities

### Main office

Address: Anonymous Ltd., Airport Road, 01530 Vantaa.

- hired Airport properties (LAK) From the Corporation

- Facilities locked and under access control Maintenance hall premises (floor plan in the attachment folder):

- hall space 1500 m 2 ,

oh where e.g. lockable cabinet unfit for flight for parts (class 3),

- equipment room 12 m2​

- office space 23 m 2 , where also

  - goods reception point

  - unfit for flight parts and waiting periods (class 2) lockable storage cabinets

  - airworthy spare parts (class 1) lockable storage cabinets.

- in the office there is broadband connections

Workshop equipment:

- electrical network 230/400V,

- warm and cold water (in the hall 2 water point)

- compressed air station

- personal lift

#### Permanent line maintenance facilities

- permanent line maintenance point is accepted organization for the use of 1.8.4 in accordance with.

Anonyymi Oy / Helsinki-Vantaa lentoasema

- huoltokorjaamo toimii myös linjahuoltopaikkana (pohjapiirros liitekansiossa)

1.

#### Temporary line maintenance locations

Temporary line maintenance used in the short term in operation, where No be or No It is better to rely on regular service points.

List temporary from line maintenance locations maintained this manual attachment in the 5.3. Temporary line maintenance site approved for the use of the organization in accordance with 1.8.4.

Temporary line maintenance point are occupied if necessary, run there is maximum 4 weeks.

#### Line maintenance location seal of approval and use

- Commissioning inspection

The technical director is responsible for the commissioning inspection and approval for use of the line maintenance site. In the commissioning inspection are being investigated place technical and practical suitability for the intended scope of work and, if necessary, site instructions will be prepared.

- Approval qualifications:

  - in hand maintenance work required literature, tools, maintenance equipment and spare parts,

  - if necessary electricity, water, compressed air and communication links,

  - in hand valid hall space or conditions enable working outdoors,

  - filled following qualifications:

oh work item can be protected from the rain, from dust and from the wind,

oh on the spot there is sufficient lighting and decent working conditions,

oh work can be interrupt, if circumstances deterioration jeopardize work quality,

oh conditions recorded to the maintenance location report ( ref . 2.13.5),

  - welfare can be make repair shop maintenance standards by.

- Seal of approval

  - permanent line maintenance point or its change is accepted With NAA 1.11 in accordance with.

To the amendment is attached maintenance point commissioning inspection minutes.

  - The temporary line maintenance site is approved for use by the technical director through an indirect approval procedure in accordance with section 1.11. The notification of commissioning shall be accompanied by a description of the maintenance site. commissioning inspection minutes and copy updated MOE.n point 5.3 pages. Service point can be take for the use of, when NAA there is given confirmation (e.g. (by e-mail) upon receipt of the service location's approval notification.

- Use

Use of the venue and orientation training will be agreed upon and arranged as necessary. with the operator.

Before every one use ensure, that place there is acceptable okay intended for maintenance.

During maintenance, it is checked that: conditions meet the requirements and The condition information is recorded in the service station report.

New a permanent maintenance site is attached to the organization The quality manager decides on the audit of a temporary maintenance site .

## ORGANIZATION MAINTENANCE PREPARATIONS AND WORK SCOPE

#### Organization maintenance capabilities and their determination

Maintenance capability refers to the ability of a maintenance organization to perform aircraft or equipment maintenance in accordance with the approved maintenance requirements. Maintenance capability is defined and maintained through maintenance capability inspections according to the scope of work.

### Aircraft and equipment maintenance in maintenance readiness review it is stated, that:

- The maintenance requirements required for the approved scope of work are specified and can be filled.

- Maintenance personnel HTA- and task authorizations are sufficient and in effect.

- Maintenance information required by scope of work (AMC 145.A.45b) are available and up-to- date and cover the requirements of maintenance programs, TC/STC holders and aviation authorities. In addition, additional type-specific CDCCL and EWIS requirements are clarified and, if necessary, methods in accordance with the requirements are developed.

- Rental tool rental agreements have been made and each The tool procurement procedure has been defined (ensuring availability).

- Frequently needed equipment, small spare parts and general supplies, as well as standard spare parts, oils, greases and fluids, have been defined and purchased in stock.

- Status-, circumstantial and special requirements (incl. spare parts storage and handling) can be to do according to the requirements.

- The control targets required for maintaining capabilities have been defined. and the monitoring procedure identifies and reports the expiry of that capability.

In maintaining capabilities, it is ensured that: maintenance capabilities correspond to the scope of work of the operating license and that capabilities temporary lack of identified and are taken into account work orders in approval.

Maintenance readiness review does machine type HTA. Inspection results with references is recorded in a review document, which is stored. The review is approved by the technical director.

The maintenance readiness of machines under maintenance contracts is maintained continuously. The maintenance readiness of machines that are occasionally serviced is activated at the latest when the work order is approved. In addition, a maintenance readiness review is carried out or checked if

- work to the extent is being sought change, new machine type or equipment maintenance,

- aircraft or device maintenance program or instructions comes significant change,

- in maintenance readiness is detected change- or need for update.

Provided something like that paragraph influence maintenance readiness, so technical manager does they measures, which the required readiness is achieved. The missing resource or material (personnel, training, facilities, material, work equipment, spare parts or the like) is acquired or its availability is ensured through rental or loan agreements. If the deficiency cannot be filled, the technical director makes the necessary changes to the organization work to the extent or negotiates substitute procedure NAA with.

If maintenance readiness comes temporary lack of for a fixed period, so scope of work will be limited for a corresponding period. The restriction and its removal, together with the reasons, are recorded in the maintenance readiness document.

#### Organization work scope

The maintenance organization can perform maintenance in accordance with its operating license and issue a certificate of release to service in accordance with Part-145 standards for the aircraft and equipment mentioned below in accordance with the scheduled maintenance or mission restrictions mentioned in the relevant section.

### Under maintenance mean here in connection with what any next work or their combination:

- deadline-, calendar time or one-time use maintenance or inspection (points 1 and 2),

- troubleshooting and -repair, version/device/part exchange and composition,

- renovations and repairs 2.12 and 2.9 according to on conditions and with restrictions,

- special maintenance with conditions/restrictions according to 2.24 (permitted penetrant dye inspections , borescope - inspections, knock tests of laminate structures, weighing).

### Workshop-, line- and equipment maintenance

A1/A2 Airplanes

| Class | Manufacturer and type | Line maintenance | Workshop services |
| --- | --- | --- | --- |
| A2 other than require motor - powered aircraft | Cessna 152 | 50 and 100 fh | Annual |
|  | PA-28 Series | 50 and 100 fh | 500 fh , 1000 fh , Annual |
| A2 require motor-powered aircraft | Beech 90 Series (PWC PT6) | All | Phase 1 - 4 |
|  | Cessna 525/525A/525B (Williams FJ 44) | All | All |
| A1 Motor-powered aircraft | Bombardier CL-600- 2B16 | 6 month / 12 months / 400fh | - |

C Equipment maintenance

Equipment maintenance comprise following Scope of work:

C14 Landing gear

Detailed description The scope of work for equipment maintenance is presented in a separate MOE In the Component Capability list.

### Line maintenance

Line maintenance are limited AMC 145.10 point 1 (Line Maintenance​ allowed for line maintenance work, and is defined on a case-by-case basis in the approval procedure in section 1.8.4.

## CHANGES NOTIFICATION

Maintenance organization inform For NAA if:

- organization name, domicile or locations changing,

- responsible manager, named responsible persons or quality manager changes,

- in the premises, equipment, tools, materials, methods, scope of work or certifying staff that may affect the operating permit.

License enlargement will be announced on the form EASA Form 2 (or with the corresponding NAA form). Change of person in charge will be announced on the form EASA Form 4 (or with the corresponding NAA form).

The above-mentioned changes and the corresponding procedures will be negotiated before the change is implemented or if it is No be possible or open, so as much as possible soon. About the procedure answer technical the manager, who must also take into account the indirect effects on the maintenance organization, manual and maintenance operations when assessing the change.

If an unplanned change occurs in the above matters (e.g. as a result of an unforeseen event), the responsible management of the organization will ensure that a temporary operating model is agreed with NAA before the new procedure or person is approved by the authority. The actual notification will be made in the manner required by NAA .

If the organization ceases operations, the agreed maintenance documents will be the responsibility of the maintenance organization. (2.17) will be handed over aircraft to the owner or For the CAM organization. If If the owner cannot be traced, the maintenance documents are kept as required by NAA .

### Part-145 requirements implementation

The Technical Director monitors the implementation of Part-145 and, in relevant parts , Part -M and Part-66 regulations on a monthly basis. changes EASA and NAA from internet sites. If above. Part requirements changes, a written impact assessment is carried out for the changed items, stating the changes points/issues mixed measures new/changed requirements to fulfill or that no changes are needed.

If the impact assessment results in a change to the MOE , the assessment will be included in the MOE amendment submitted to the NAA . Admitted change are taken for the use of, when implementation and possible The additional measures required by NAA have been taken.

About the implementation procedure answer technical manager.

## ORGANIZATION MANUAL AMENDMENT PROCEDURE

### Starting points:

The header of the manual page contains the page number (control identifier), publication date and revision level. The manual will not be modified by manual corrections, nor will temporary changes be made to it.

Manual changes is accepted With NAA before their implementation. From the previous by way of exception The changes mentioned below can be approved by the indirect approval method. In this case, the change is approved by the Technical Director and the change is notified to the NAA . The NAA may return the change for re-drafting for justified reasons.

### However indirectly by method can be accept

- manual and its attachments small bug fixes and text clarifications,

- temporary line maintenance locations for the use of seal of approval 1.8.4 by,

- HTA staff authorizations 3.4 by,

- attachments and lists (part 5) updates.

### Change procedure:

Manual changed text or picture (deletion or addition) is marked in question point left side with a vertical line. In a change, all pages receive the same change number.

When handbook there is changed, so technical manager book manual inspection in effect to the list of pages available (page 5).

The change documents are sent to NAA electronically. The documents are provided with appropriate signatures and an accompanying change description is attached.

Changes seal of approval:

When the change is approved by NAA, then technical the leader sends new change electronically to NAA , which, after inspections and possible additional clarifications, will notify you in writing of the approval (or conditional approval) of the change.

When the change is approved by the technical director (implicit approval), so he records the approval in the change list (page 4) and sends the new change to NAA electronically. The change can be implemented, when NAA there is given confirmation (e.g. by email) change notification from the reception.

When change there is admitted, so technical manager

- update change list (if necessary),

- answer distribution according to manuals updating (excl. (NAA)

  - update self or delegate update in writing with instructions/confirmation manual for holders,

- answer change commissioning requiring about training and information,

- archive old master song at least 3 for a year and dispose of other old changes.

### Manual (MOE) distribution:

- Technical manager ( master song )

- Aviation Authority (electric version)

- Personnel (electric version)

### Responsibilities:

From the manual and its for in step with hold, about changing, distribution and archiving answer Technical Director. He is also responsible for the supervision and maintenance of forms related to manual methods. The forms in use, along with their revision information, are presented in a summary form list.

# PART 2: MAINTENANCE METHODS

## SUPPLIER AND SUBCONTRACTORS JUDGMENT AND CONTROL METHOD

The maintenance organization uses EASA-approved organizations or aircraft/component manufacturers as its suppliers. Also other so-called intermediary organizations can be used if the EASA acceptability of the item can be ensure reliable way. Of possibilities by Standard suppliers are used, for which a separate supplier list is maintained. The reliability and quality of suppliers are monitored in connection with deliveries. If problems arise with deliveries (goods or documents), then thing are being advertised. Provided recurring problems appears, then supplier are exchanged.

Maintenance organization No use subcontractors.

## AIRCRAFT PARTS, DEVICES AND MATERIALS INSPECTION AND SEAL OF APPROVAL

For the incoming for the goods (part/device/material) will be done reception inspection, where it is stated:

- shipping package there is proper and transport damage No be,

- packaging contents answer order and packing list,

- required airworthiness documents are with,

- goods and documents identification information markings corresponding each other,

- in visual inspection by goods with protection there is okay.

- The acceptance inspection of ESDS devices is performed on a grounded table or platform. The device handlers use a grounding wrist strap if necessary. After the inspection, the device is stored in its original protective bag until it is installed.

The receiving inspection and airworthiness approval* of the goods is carried out by the person issuing the release certificate. After airworthiness approval and inventory records, the parts/equipment/material can be installed on the aircraft or stored.

If the goods are found to be defective upon receipt, it will be suspended and the goods will be returned to the supplier (or destroyed). If the airworthiness documents for the goods are incomplete, the missing documents will be obtained. before goods approval to the warehouse or use. If documents No get, so The goods are returned to the supplier. Goods whose receipt inspection has been interrupted for any reason are handled and stored in accordance with sections 2.3.2-3.

### *Airworthiness seal of approval

- New part or device can be accept, if for it there is published one next document:

  - EASA/JAA Form 1 (ICE before 28.11.2004),

  - FAA Form 8130-3 or 8130-4, manufacturer or FAA representative written,

    - Restriction: The critical part No replace with PMA part.

  - TCA Form 24-0078, manufacturer or TCA representative written,

  - Original Manufacturer's Certificate, original manufacturer certificate ,

  - EASA or NAA separately approved documentation.

- Used part or device can be accept, if for it there is published one next document:

  - EASA/JAA Form 1, EASA/JAA organization written by (ICE before 28.9.2004),

  - FAA Form 8130-3 or 8130-4, EASA/JAA approved FAA organization written,

  - TCA Form 24-0078, EASA/JAA approved TCA organization written,

  - EASA or NAA separately approved documentation.

- Standard part and material can be accept in the following in cases:

  - assemblies there is defined standard parts aircraft in the parts list (part number refers to to an approved aviation standard (e.g. AN, MS, NAS).

  - part listed aircraft in the standard parts list or in other manufacturer in the publication,

  - material (e.g. aluminum sheet, fat) own approved aviation specification.

The above parts require a manufacturer's declaration that the parts comply with the above standards. If the parts in question come from a separate supplier, they must contain identification information and a declaration from the supplier that the parts comply with the information provided and that the original airworthiness documents for the parts are available if necessary.

## AIRCRAFT PARTS, DEVICES AND MATERIALS POSSESSION

#### Rating

Aircraft assemblies, raw materials and supplies classified as follows:

Category 1: Serviceable parts (with appropriate airworthiness documentation). Category 2: Unusable parts (repairable)

Class 3: Unrepairable assemblies (usability No Oh return)

Class 4: Standard parts, that are determined manufacturer in service information or in the parts list

Class 5: General parts, supplies and raw materials whose origin is known and has been found to meet aviation requirements.

#### Marking

When receiving usable parts, an incoming batch identification number is attached to the incoming batch EASA/JAA Form 1 and parts, which is used to identify the order number of the batch (and, if applicable, the arrival date). In addition, expiring material is provided with a “EXPECTED TO USE” label. After that, the material control files are updated and the documents are archived, and the material is stored or put into service.

A usable part (class 1) can also be equipped with a green escort card, if necessary, on which the part identification and monitoring data is recorded. Provided part in airworthiness documents there is shortcomings, so they are clarified before the part is accepted into category 1.

Unserviceable (class 2) parts are stored separately from serviceable parts and, if necessary, marked with a red with a farewell card, to which part identification and monitoring data is recorded. Over here to class This includes parts undergoing maintenance, repair or modification, as well as parts that are in the acceptance inspection or subject to airworthiness certification.

A part that is unusable (category 3) is always stored separately from other parts. The parts are handled according to 2.3.5. Category 3 includes a spare part

      - which there is achieved or exceeded it approved lifetime, or

      - where there is unrepaired fault or damage, or

      - which there is been in an accident, which likely influence its airworthiness

      - whom no airworthiness be note or may into force by the manufacturer or authorized by the guardian (responsible).

#### Storage

Each part of the spare parts group (categories 1, 2 and 3) is stored in its own lockable cabinets. The locations of the storage cabinets are described in the floor plans of the permanent maintenance locations (1.8.1-2). Each storage space marked their classification in accordance with. For temporary use to service points No store spare parts. All ESDS equipment or parts are stored in an appropriate in a protective bag.

Standard parts (class 4) and general materials (class 5) recorded recognizably and preserved separately in appropriate storage locations (warehouses or work stations). Storage is carried out by HTA.

Technical manager answer, that

      - materials, parts and equipment service life monitored manually,

      - equipment change- and airworthiness directive work will be done if necessary,

      - outdated material will be removed from stock or from use mixed destroyed,

      - storage conditions maintained manufacturer standard fulfilling.

The storage facilities are kept locked or manned. Only a permanent employee of the maintenance organization is authorized to enter the storage. personnel, other persons under the supervision of a permanent person.

#### Delivery maintenance operations

The identification number of the material, part or device released from the warehouse is recorded in the spare parts specification for the maintenance work in question and on the device card (if it is opened).

Before commissioning, the HTA checks that the material, part or device is serviceable in terms of airworthiness documents, service life limitations and storage method. In addition, before installing the component, it must be ensured that the different modification levels are compatible and that the relevant airworthiness regulations and appropriate acceptance inspection have been carried out. If this If it is found at this stage that the device (or similar) cannot be installed, it is moved to the non-flightable parts (class 2) cabinet for further action.

Equipment and components of aircraft under maintenance are stored in a supervised or locked location.

#### Scrapping ( mutilation )

A part or device that cannot be repaired is handed over to the operator, properly marked. The part/device can be also scrap operator assignment, scrapping will be done with suitable method, for example

      - by grinding

      - by burning

      - by removing lug or equivalent feature

      - by doing permanent transformation

      - by doing hole by flame cutting or with a chop saw

      - by melting

      - by cutting into pieces

      - other authority approved method.

A part or device that is beyond repair may also be transferred, with the permission of its owner, to an appropriate and reliable organization for educational or research use, provided that the part/device is marked and a written transfer agreement is signed that the part/device may not be transferred further. In such cases, the part/device may be transferred using the following procedures:

      - are equipped permanent "NOT" SERVICEABLE” with a label

      - by removing part originals identification information

      - by removing identification plate

      - by maintaining a serial number and unique identifier-based tracking and traceability records for unrepaired parts

      - by giving written instructions part final scrapping.

## TOOLS AND EQUIPMENT ADOPTION

#### General

The technical director is responsible for the procurement, approval and maintenance of the organization's tools, measuring instruments and equipment. He or she makes tool purchases in such a way that they meet the requirements of the planned maintenance activities in terms of quantity and quality.

All tools and equipment (individually or in groups) are recorded and monitored. A file is kept of special tools, which contains all special tools used by the maintenance organization. The technical manager is responsible for monitoring.

Rarely, a tool that is needed can also be acquired by renting. In this case, the use of the tool is agreed upon in a rental agreement. The place and procedure for acquiring the rental tool are recorded in the maintenance readiness review. The technical director is responsible for the procedure.

#### Substitute tool/equipment seal of approval

If manufacturer tool or equipment No be within limits available, so its in place of can be obtain a replacement tool or equipment that can achieve the required maintenance result. The original model approved for use under Part-145 or the manufacturer's working drawings are used as an aid to comparison. A document is prepared on the approval of the replacement tool/equipment, which records the basis for substitution, the comparison procedure and the intended uses.

A device card is opened for the replacement tool/equipment, where the approval and the original tool number as well as a reference to the review document are noted. The measures are carried out by the HTA. The card is kept in the TOOLS folder.

#### Tools/equipment manufacturing and seal of approval

If the tool/equipment is not reasonably available, it can be manufactured using the manufacturer's published specifications. drawings and instructions in accordance with. Technical manager does before in a manner consistent with the principles of 2.4.2 of the commissioning review of a self-made tool. Self-made tools are numbered manufacturer with tool numbering and organization with an identifier. Manufacturing documents are archived in the TOOLS folder. The actions are carried out by HTA.

## TOOLS AND EQUIPMENT MAINTENANCE AND CALIBRATION

#### Tools and instruments registration and marking

A register is kept of the maintenance organization's maintenance and work equipment, which shows the identification number, name, location, intended use and any maintenance requirement (or its reference).

Separately supervised tools or equipment to be registered time control mixed are equipped with your own device card. In addition to identification information, the device card records the maintenance/calibration period and the performance and authors of the maintenance/calibration period. The device card (including the last maintenance/calibration report) is stored in the TOOLS folder. The time control list shows the last and next maintenance/calibration period in addition to the monitoring period. A sticker is attached to the monitored tools and equipment, indicating the expiration date of the maintenance/calibration.

General tools and -equipment are being checked (and if necessary being maintained) annually. Inspection is recorded.

The technical director is responsible for the registration, marking and accounting of tools and equipment, as well as for supervision, inspections and maintenance.

#### Calibrated measuring instruments control

Measuring devices, from whom used for adjustments or for maintenance measurements, calibrated in question device manufacturer or in accordance with the recommendations of the aircraft maintenance instructions. In the absence of a recommendation, the calibration period of the measuring device there is one year that experiences may be extended for up to three years.

Based on experience , it means that the measurement results obtained with the measuring device have been found to be reliable and there is no malfunctions have occurred and that the same measurement result is repeatedly obtained in comparison measurements. The recommendations given may be deviated from if the structural measurement accuracy of the measuring device is multiple of the required one and a replacement recommendation is received from the calibrator of the measuring device.

The decision on the extension period may also take into account the low level of use and storage conditions of the measuring device (no changes). However, the calibration periods of measuring instruments in daily use will not be extended.

Decision to continue will be done measuring device annual review in connection with. Technical manager Oh grant a continuation maximum of two times. Extension period The approval, including the reasons, is recorded on the equipment card.

If it is suspected that a measuring device does not meet the requirements, it is calibrated before use, regardless of the inspection period. Measuring devices used for process control are calibrated only when necessary. Measuring devices to be calibrated are marked with the “NEXT INSP.” information and other measuring devices with the “NOT CALIBRATED” label. Measuring device groups are also distinguished in accounting (control).

The technical director is responsible for supervising the measuring instruments to be calibrated and their delivery to an approved measuring instrument inspection organization. Calibration is carried out in an accredited measuring instrument laboratory. or with the supplier, whom measurement methods can be traceable to national standards.

#### Measuring instruments seal of approval for the use of

New or calibration came measuring instrument are being checked and is accepted for the use of, if it fill The technical director and the instrument's owner are responsible for the commissioning inspection and approval for use . technical inspection does HTA. Inspection and for the use of seal of approval recorded in question on the tool's device card and the measuring instrument is equipped with a control label.

During commissioning inspection it is stated, that measuring device calibration there is made duly and its result there is acceptable mixed calibration tables (resp.) are usable. If necessary can be done complementary correction table, provided error magnitude influence measurement accuracy. When is the measuring instrument there is calibrated or admitted for use only in the limited in operation or in the measurement range, then it is clearly marked on the correction table or on the tool.

Provided appears, that new measuring instrument No be calibrated or it there is made intended inadequate for the intended purpose, the measuring device is calibrated before use.

## TOOLS AND EQUIPMENT USE (ML. ALTERNATIVES TOOLS)

General tools are placed in tool trolleys and maintenance stations. The tool user inspects the tools on the trolley or workstation at the end of the working day and after the maintenance work is completed. Deficiencies are identified immediately. If it is determined that the condition of the tool is not appropriate, the observer remove tool The Technical Director is responsible for substitute tool about the acquisition.

Special tools and other shared equipment are stored in tool cabinets or workstations where they can be used. there is own place. Tool/equipment fitness are being checked always commissioning in connection with and it is returned to storage in working order after the need for use has ended.

Special tools in their entirety and general tools on a random basis are inspected by the technical director monthly, and the inspections and observations are recorded.

Defective or suspected defective tools are removed from use. The tool is sent for repair or a replacement tool is purchased.

If necessary, training in the use of special tools is initially provided during commissioning and subsequently in recurrent training. The training is recorded in the training records.

The technical director is responsible for the above procedures, and is also responsible for approving the alternative tool.

## SERVICE FACILITIES CLEANLINESS CLASSIFICATION

The maintenance organization does not perform any maintenance work that imposes special requirements regarding cleanliness, such as instrument repairs.

The cleanliness of the premises is maintained such that any maintenance and equipment and tool work that is required by the scope of work can be carried out without compromising the quality level. If work is interrupted, the maintenance items and removed parts are appropriately protected.

The premises are cleaned when necessary. The service point is always cleaned after maintenance. The technical director is responsible for ensuring a sufficient level of cleanliness.

## MAINTENANCE INSTRUCTIONS, THEIR FOR IN STEP WITH HOLDING AND USABILITY MIXED CONTACT FOR TC HOLDERS INSTRUCTIONS

The maintenance instructions used are those specified in the aircraft maintenance programs or maintenance work orders, which is acquired or made available to the maintenance organization as follows:

- The operator's CAM organization, when the maintenance contract has agreed to provide the maintenance instructions (maintenance information) required for the implementation of the maintenance program to the maintainer. The instructions are made available as written or electronic publications or Through the Internet.

If maintenance instructions are relied upon that are not specified in the approved maintenance program, then their comes be equipped operator CAM organization in writing with insurance, that The instructions in question comply with the requirements and are up to date.

- By subscription agreements from TC/STC holders or approved literature suppliers, when the maintenance organization is responsible for purchasing maintenance instructions. The instructions are made available by post or via the Internet.

Unless from maintenance literature found ready maintenance instructions or -lists some work to do, so For the work , instructions or a list are prepared in accordance with the manufacturer's requirements, which include the reference information and change level of the original publication, as well as the technical director's approval.

Maintenance instructions or them specifying information are also airworthiness regulations (more specifically 2.11) and EASA Emergency Conformity Information (ECI) and Safety Information Notices (SIN) publications that are monitored monthly for parts that affect the scope of work (internet).

Maintenance instructions and literature are kept in the workshop office, where they are available to technical personnel. in hand. Technical literature, maintenance instructions and Future changes and additions to the airworthiness regulations are collectively processed by the technical director, who decides on the necessary further measures (distribution, information, updating of time control, etc.).

Unapproved literature will be marked “FOR EDUCATIONAL USE ONLY” or “NOT REVISED”. Working copies will be marked “WORKING COPY” and destroyed after use. The technical director is responsible for the procedure.

Maintenance instructions for the use of approval and supervision answer technical manager. Keeping maintenance instructions up to date is included as one point for maintaining service capabilities (see 1.9.1 for more details).

## REPAIR METHODS

#### Defects repairs

The operator's CAM organization is primarily responsible for analyzing a fault or malfunction in an operating aircraft. If the analysis is assigned to a maintenance organization by work order, it is performed by the aircraft type. HTA under. If necessary he give statement machine airworthiness or further action to the operator. If a diversion flight is required, the operator obtains permission to fly from the NAA . The fault is repaired under approved conditions according to the work order in the manner of workshop maintenance . If repairs have to be made under exceptional circumstances, they can be done with the operator's permission, the repair procedure is approved (2.13.4) by the technical director.

The HTA of the machine type in question will take a position on any fault found during maintenance (see 2.15 for more details). Any significant repair needs will be reported to the operator immediately. The manufacturer's published fault finding information will be used in the search for faults. defects localization methods and repair will be done workshop maintenance standards in accordance with.

Transferred defects: If a defect cannot be repaired, it may be transferred with the consent of the operator for a valid reason. However, the transfer of the defect must not conflict with the aircraft's Minimum Equipment List (MEL) or maintenance program. The transfer of the defect shall be made in accordance with the procedures specified by the operator's CAME. in accordance with to the maintenance certificate and deferred defects (resp.) -sideways. If above. documents are not in use, then the fault transfer review is carried out in accordance with the TC holder's maintenance instructions and minimum equipment requirements by and recorded in the trip log. Troubleshooting is given deadline (date or month ), which is recorded in the maintenance certificate. The fault is transferred by HTA.

Recurring faults: If a fault begins to recur more frequently than previous experience or information suggests, the operator is notified and the cause of the fault is thoroughly investigated. If necessary, the manufacturer's service support is used.

#### Damage repairs

If the organization's task (work order) is to repair damage, it will begin with a preliminary inspection, which does machine type HTA. Under review to be clarified to damage led event circumstantial and situational factors, the damage caused and foreseeable repair measures. The inspection will be prepared report, where above. information in addition is presented possible further research needs mixed Clarifications specifically requested by NAA , the insurance company or the operator.

### Repair as maintenance work

If the inspection determines that the damaged item can be restored to its original condition by repairing/refurbishing the part(s) and/or replacing them, then the repair is considered maintenance work, which the organization can do scope of work aircraft in accordance with the following terms and conditions when filled:

      - damage can be verify bordering exchangeable or to be repaired to the sub-set,

      - repair can be make TC/STC holder instructions by free fixed intended demolition of primary structures,

      - available with instructions there is Part-21: adequate seal of approval,

      - repair after airworthiness can be to make sure unambiguously.

## AIRCRAFT MAINTENANCE PROGRAM COMPLIANCE

Aircraft maintenance will be done operator work order and machine maintenance program in accordance with. If the maintenance program No no, so are followed TC/STC holder published maintenance requirements and maintenance bulletins.

Compliance with the maintenance program for individual maintenance is ensured by maintenance planning (2.28) and is checked in the procedure for issuing a maintenance certificate, which also determines the next scheduled maintenance according to the maintenance system and its deadline, as well as the deadlines for maintenance measures to be carried out before the next scheduled maintenance and for the repair of defects that have been postponed.

If tasks related to the maintenance of the operator's maintenance program or maintenance supervision are agreed to be the responsibility of the maintenance organization, this will be agreed separately in accordance with the NAA requirements . way.

## MANDATORY AIRWORTHINESS REGULATIONS CONCERNING PROCEDURES

The maintenance organization is responsible for the maintenance capabilities required to perform airworthiness directives for aircraft within its scope of work, which are checked and updated as part of the maintenance capabilities (1.9.1).

The maintenance organization obtains or receives from the CAM organization the airworthiness directives ( ADs ) and Emergency Conformity Information (ECI) and Safety Information Bulletins (SIB) publications. Own procurement is done via the Internet from the aviation authorities of the TC holder countries as well as from EASA and NAA . For maintenance contract aircraft, the impact of regulations on maintenance readiness and -activity are being checked and updated monthly. Others machines For AD maintenance readiness, the review is carried out at the latest upon approval of the work order.

Airworthiness directives are carried out based on the operator's work order and maintenance data provided . The performance is recorded in the maintenance work documents as implemented and presented in the maintenance handover. Surveillance data about the update answer operator. If maintenance in connection with If an unfulfilled airworthiness directive is identified, the operator will be notified, who will decide to attach it to the work order.

About the procedure answer technical manager.

## VOLUNTEERS CHANGES CONCERNING PROCEDURES

If a maintenance organisation is required to carry out a voluntary change work published by a TC/STC holder, the prerequisites for carrying it out are first checked. The check determines the adequacy of the scope of work, resources and approved instructions, and determines whether the change work can be carried out by decision of the technical director or whether it requires EASA approval. The performance assessment of the change work is made by EASA or Part-21 approved design organisation. information on the basis of. In unclear in cases The matter will be clarified with the NAA .

If permitted by the above inspection and modification instructions, the modification work published in the maintenance bulletin can be carried out directly by decision of the technical director, if the publisher of the bulletin is the type approval holder of the machine or device, the authority of the type approval country, EASA or an entity approved by EASA .

In others in cases renovation work can be make only EASA with permission. If permit application comes task of the maintenance organization, then a work plan (including modification instructions) is attached to it. The work plan is presented modification work achievement step by step, usable instructions and work methods and procedures for ensuring airworthiness. The operator applies for a modification work permit. The modification work is carried out in accordance with the approved work plan, modification work instructions and permit conditions in accordance with.

EASA approved STC modification work can be make directly technical leader by decision. Similarly, STC modification work approved in an EU member state before 28 September 2003 can be carried out without EASA approval if the scope of work of the maintenance organisation allows it, however, the modification work and its approval reference must be notified to the NAA before implementation.

## MAINTENANCE DESIGN, WORKING DOCUMENTS AND WORK STAGES

#### Maintenance working documents

- Standard forms: Work order, maintenance certificate, work- and checklist and spare parts specification

- or above. forms with a substitute aircraft monitoring program with form printout.

- Maintenance lists, used maintenance scope by,

- Additional forms, used if necessary.

Working document pages are forms, printouts or copies of maintenance instructions. A list of the forms to be used and samples are in the form folder. Approval and control of the use of forms answer technical manager. Additional forms can be used: also forms published by the aviation authority. For very minor maintenance work and repairs, the standard forms can be replaced with a single form containing the relevant matters.

The work order is placed on a form prepared by the operator or maintenance person. The order (and order details) are signed by both parties. When separately arranging for small CAMO orders or recurring work can be order and to administer with a monthly work order, here if necessary The CRS is entered in the logbook/technical logbook.

#### Working documents filling

Work preparation in connection with working documents recorded maintenance scope required basic information. During maintenance work phases, the performed measures, inspections, measurements and their results are recorded in the documents. The measurement value is recorded in full.

Done maintenance work to acknowledge technical leader accepting in extent mechanic, mechanic or HTA. Set-off rights is defined technical leader maintaining on the list. In others In some cases, double acknowledgement is used, and verification is done by HTA. The acknowledgement is made with initials or a stamp. Marking there is to be very readable and undeleted. Incorrect entry can be crossed out depreciation basis or substitute entry through. Non-regarding maintenance list point acknowledge HTA with N/A and initials.

The certificate of release to service is issued by an authorized HTA on behalf of the maintenance organization. The acknowledgement consists of the date and signature and authorization number or stamp. In use The stamps and personal authorization numbers (ID) and initials used as acknowledgements are presented in the personnel register.

#### Maintenance design

Technical leader named HTA plans maintenance work order by as follows:

      - by ensuring that the maintenance capabilities required by the work order/maintenance in question are sufficient and valid and that the ordered maintenance can be completed within the scope of the maintenance organization. Other Part-145 suppliers or the use of subcontractors is reported to the operator,

      - ensuring that all work planned for maintenance is included in the operator's work order and that the parties have a common understanding maintenance according to the work order coverage,

      - by giving the maintenance a job number, recording basic maintenance information on standard forms and compiling the work documents in a maintenance folder,

      - by ensuring the availability of special tools, spare parts, materials and instructions, as well as the level of maintenance conditions and the availability of personnel for the maintenance work in question,

      - by phasing maintenance into the available workforce, space and equipment resources, taking into account any special work/maintenance and possible human author limitations,

      - by anticipating additional inspection items for critical jobs and adding them to maintenance lists or work and inspection lists,

      - by taking into account CDCCL and EWIS methods instructions and inspections, if them there is published.

#### Maintenance work stages and entries

| Operation | Factor | Posting procedures |
| --- | --- | --- |
| Maintenance design | - HTA / HTA under | - more specifically in the previous in the song (2.13.3). |
| Machine reception | - HTA or named mechanic - operator representative | - are being checked in the planning phase made working documents - will be done for the machine income tax and recorded additional observations - ensure consensus from a work order content and coverage |
| Maintenance work | - HTA - mechanic/installer | - acknowledged made work maintenance-/ work- and checklists - recorded at work used spare parts spare parts specification list |
| Inspections | - HTA | - are being checked and are verified double-check items - will be done critical work additional checks and - acknowledgements - will be done regional inspections (2.25) |
| Deviations - reportage - notification | - observer - HTA | - deviations and other observations recorded maintenance documents and analyzed in a documented manner in the internal reporting system (2.18) - a serious deviation is reported to the authority within 72 hours, to the manufacturer and for the operator (2.18) - other safety observations processed volunteer notification procedure in accordance with (2.18) |
| Trial run | - named test user - HTA or named mechanic | - record the test operation values and adjustments made in the test operation report or work- and checklist |
| Shakedown flight | - test pilot - HTA or named mechanic | - recorded score test flight report (if necessary) - will be done test flight caused work and inspections |
| Airworthiness assessment | - HTA | - will be done machine/device final inspection - are being checked maintenance documentation and their entries - filled technical diary and its maintenance- and device cards - will be acquired operator seal of approval for the transferred for work |
| Service certificate and machine delivery | - HTA - operator representative | - it is stated work order execution status - will be done additional work and processed transferred work - processed deviation observations and their notification - it is stated machine airworthiness status - are assembled and are numbered maintenance document package - is given maintenance certificate |
| Distribution of maintenance information and recording | - HTA | - are recorded maintenance documents - delivered maintenance documents For CAM organization ( resp.) |
| If for maintenance participates trainee, so he works always named director under the supervision of. |  |  |

#### Approved maintenance points outside task random work

Necessary maintenance work: If the aircraft is not airworthy and the repair of the defect is attempted on site, the technical director will determine the repair conditions on site and make a plan for the repair work. and its demanding procedures mixed book repair work required instructions, tools and personnel and decides on the repair work. The same procedure is followed for the airworthiness directives to be made before the next flight. and maintenance bulletins in performance or operator separately in cases of airworthiness verification ordered.

Individual work phases: Maintenance, paintings, composite and metalwork individual work phases can be carried out outside the workshop if this is justified in terms of occupational or environmental safety and work quality level can be retain. Operation, its place of performance and conditions recorded in the maintenance site report. The technical manager is responsible for the procedure.

### Service location report:

An acceptance and operational review is conducted for each necessary maintenance work and individual work step carried out at a temporary or approved maintenance location or facility . The review is recorded. to the service location report, which The technical director is responsible for the procedure.

## TECHNICAL ACCOUNTING CONTROL

Before issuing a maintenance certificate, the person issuing the maintenance certificate must ensure that all the maintenance in question is maintenance procedures has been done out and recorded in the maintenance records. Maintenance to end up with A volume bundle is compiled from the maintenance documents, the content of which (monitoring information) is shown on the maintenance certificate or cover page, to the volume bundle is attached also components airworthiness documents. Maintenance documents that are not to be retained will be destroyed.

Maintenance documents will be handed over or sent operator For CAM organization maintenance after (2.17). Before handover, the maintenance organization takes written copies or electronic records of the documents, which are stored in secure databases (2.21). Written documents are stored in a locked metal cabinet in machine-specific folders in the technical office. The technical manager is responsible for the proper storage of technical records and for protecting them from damage, alteration and theft.

Maintenance document files preserved at least three years thence from the date, when The aircraft or part thereof has been certified as airworthy and has left the maintenance organization.

Lost document in replacement we act case by case NAA instructions by.

## MAINTENANCE IN CONNECTION WITH OF THE OBSERVED DEFECTS REPAIR OR MOVE

Detected defects are recorded in the work and inspection list and attempts are made to correct them during the maintenance. If the defect repairing not be from somewhere justly be possible, so its move is handled as follows:

### Airworthiness discouraging fault:

Airworthiness discouraging fault comes fix before next flight. Repairing waiting The aircraft keys and the journey log are kept in locked maintenance facilities.

### Airworthiness limitedly affect fault:

Fault can be transfer operator CAME according to procedures. Moved fault recorded maintenance documents to the maintenance certificate and travel diary deferred defects -sideways (resp.) equipped With MEL reference or elsewhere approved on a transfer basis. If CAME- and MEL instructions No be, so If necessary, fault transfer is carried out according to the TC holder's maintenance instructions and minimum equipment requirements. In all cases possible operating and Repair time restrictions are recorded in the trip log.

Possible transfer flight may be carried out if permitted by the MEL or minimum equipment requirements or, if the above is not possible, by the operator's CAME according to procedures or NAA with permission.

When the transferred fault is repaired, a maintenance certificate is issued for the work and the transfer is acknowledged as " deferred ". defect ” page (resp.) or travel diary.

### Airworthiness unaffected fault :

Move can be make operator with permission and will be done CAME (or TC holder) according to procedures.

### Entries and control

The fault transfer is made by the HTA of the machine type, the procedure with references is recorded in the transfer decision. Transferred faults are recorded to the maintenance certificate and processed maintenance handover in connection with maintenance with the recipient. The combined effect of the transferred defect or defects is assessed in the procedure for issuing a certificate of release to service. A certificate of release to service is not issued if it is suspected that the defect(s) endanger flight safety. The operator is responsible for monitoring and following up on the transferred defect.

## PROCEDURES MAINTENANCE CERTIFICATES FOR GIVING

#### Maintenance certificate administration for aircraft

Before issuing a certificate of release to service, the issuer must ensure that the maintenance organization has prepared its maintenance manual. procedures and the aircraft is airworthy to the extent required by the work order and maintenance requirements. Maintenance documents are reviewed and compiled into a maintenance package before being issued as a certificate of release to service. A certificate of release to service is not issued if it is suspected that the aircraft is not airworthy.

When performing workshop maintenance on complex motor-powered aircraft as defined in section 1.9.2, the certificate of release to service as a giver Oh act only accredited maintenance personnel, whereby there is appropriate rating for the aircraft type in question for both In the AML license, a C-class rating for the aircraft type in question.

### In the maintenance certificate there is next insurance:

"I assure you, that above described work, unless other be announced, there is made EASA Part-145 in accordance with and the aircraft/aircraft part/equipment is approved as airworthy in these respects.”

or equivalent in English :

“Certifies that the work specified, except flat otherwise specified, was carried strange in according to with EASA Part-145 and in respect to that work the aircraft/aircraft component is considered ready for release to service.”

The aircraft maintenance certificate is issued on the log page of the aircraft's logbook or technical logbook and on the maintenance certificate in the working documents, and in the case of an OPS operator, also on the technical logbook. " maintenance" statement ” -part. From the previous by way of exception version changes and For frequently repeated inspections, a maintenance certificate can be issued exclusively to the vehicle owner. Parallel maintenance certificates are issued by the same person.

In the maintenance certificate recorded maintenance author identification information, date and flight time mixed reference manufacturer's instructions or the aircraft maintenance program. For extensive maintenance, a summary description of the maintenance in question and a reference to the job number are sufficient. The maintenance certificate to be given on the log page is recorded in the space reserved for it (or maintenance records).

The maintenance certificate identification information must include the company name, license number and HTA endorsement. The person issuing the maintenance certificate must use their full signature and the maintenance organization's internal authorization number or stamp when endorsing the certificate.

### Suspended maintenance maintenance certificate

If the maintenance ordered by the operator cannot be completed due to a shortage of spare parts, downtime, and due to the end of the maintenance period, incomplete test operation/flight or essential flight task, then the maintenance organization can issue a maintenance certificate for insufficient maintenance under the following conditions:

- for those moving maintenance for parts there is operator written seal of approval,

- move will be done MEL's and maintenance program by or operator by acquiring NAA with permission,

- HTA there is satisfied, that in question move No endanger aircraft airworthiness,

- move with restrictions and with deadlines recorded to the maintenance certificate and to monitoring data.

Temporary maintenance certificate: If an aircraft has to be left somewhere other than the usual maintenance location due to the lack of a spare part and there is no spare part available with an appropriate maintenance certificate. In this case, a temporary maintenance certificate can be issued to the aircraft, even if the aircraft are installed useful spare part, but whereby No be appropriate The procedure must have the operator's written consent. The spare part can be used until the return to the home base, but for a maximum of 30 flight hours.

Limited HTA Authorization: The commander or flight engineer may issue a release to service certificate for such airworthiness directives that are performed repeatedly in conjunction with PFI inspections and in the directive there is specifically the said, that flight crew weather run required The HTA authorisation procedure is presented in section 3.4.1.

Single-use HTA authorization: If the aircraft has unexpectedly been grounded due to a defect at a location where there are no approved certifying staff, the contract maintenance organization may issue a single-use HTA authorization to a licensed mechanic with the corresponding type approval in accordance with the procedures in 3.4.1. The issuance shall be reported to the NAA within 7 days . Maintenance organization to inspect on the above grounds made work, when the machine arrives at the service point.

#### Maintenance certificate administration for parts, for devices or for equipment

When a detached part/device/equipment is serviced or removed (see 2.16.3) from an aircraft in serviceable condition (or its (subset) and will be handed over to be installed another maintenance organization by , then the release certificate is issued on EASA Form 1.

If above. part/device/equipment will be handed over to my own to the warehouse , so maintenance certificate can be give to the green escort card.

Aircraft maintenance in connection with for dependents for auxiliary devices No to give separate maintenance certificate, if It can be maintained in accordance with the aircraft manufacturer's maintenance instructions. A separate maintenance certificate is issued for maintenance of detached auxiliary equipment within the scope of equipment privileges.

EASA Form 1 forms use, accounting and storage answer technical manager.

#### In working order detached assemblies and equipment

Maintenance organization Oh issues a Part-145 release certificate requirements by serviced a part removed from an aircraft or subassembly in working order, for a device or equipment if:

      - with a maintenance organization there is legal remove and install in question part, device or equipment,

      - aircraft from the last from use there is under 6 month or can preserved TCH instructions in accordance with,

      - TCH instruction in accordance with in general inspection No be stated corrosion, leakage or damage,

      - from the machine's documents it can be ensured that part (resp.) has not been in an accident, incident or lightning strike that could have affected its airworthiness,

      - no fault has been reported in the aircraft's transferred faults or from the last flight that could be caused by the part in question (resp.) or the reported fault has been reliably located to another location,

      - maintenance- and change status can be sort out and information not limit airworthiness.

Part, device or equipment to the maintenance certificate and maintenance documents recorded:

      - whence and whereby during operation it there is detached from an aircraft or from the greater of the whole,

      - maintenance recorded with a label CHECKED / INSPECTED, if wider maintenance No make,

      - maintenance requirement references and control basis information,

      - airworthiness and for storage affect information and trammels.

## FOR THE OPERATOR GIVE MAINTENANCE DOCUMENTS

Maintenance after completion machine For CAM organization (or operator) will be handed over or sent:

- originals copies of the maintenance certificate, from a work order, work and from the checklist, spare parts specification and approved repair and modification work documents,

- originals songs installed equipment airworthiness documents and

- other operator separately requested maintenance documents.

Before handover maintenance organization take the above documents written copies or electronic records that are stored in secure volumes/databases.

If a separate agreement has been made with the operator, the above-mentioned maintenance documents may be stored by the custodian. The operator's maintenance documents that remain in the custodian's storage are the property of the operator and are available to him. If the storage agreement expires, the maintenance documents will be delivered to the operator or to the CAM organization notified by him.

If the maintenance organization ceases its operations, it shall provide the owner with the maintenance records of the machine for the last three years or keep them with the NAA. in an acceptable manner.

## AIRWORTHINESS OBSTACLE FOR THE CURRENT DEFECTS NOTIFICATION

### Internal notification system

All defects and maintenance observations that hinder airworthiness and endanger flight safety are reported, analyzed and are evaluated as part of maintenance organization safety- and quality activities.

Any person in the organization can report a defect observation. The analysis of the observation will determine: its risks, reason- and consequences mixed importance maintenance operations and airworthiness. The analysis is carried out in such a way that the required notifications can be made within 72 hours, if necessary. The Technical Director is responsible for the procedure.

The impact of the observation of a deviation on one's own operations and equipment, as well as possible further measures, are assessed. security meeting (more specifically 3.1.2.d). In the evaluation are taken also carry to the fact that how the hazard can be prevented or that its any recurrence will certainly be detected.

The organization's management ensures that maintenance personnel can freely and openly make observation reports and participate in their processing without fear of pressure. In addition, care is taken to ensure that personnel are informed of all reported observations and the related actions and feedback.

### Notifications aviation authorities and For TC holder

If above. in the procedure or under maintenance it is stated flight safety seriously dangerous fault or a malfunction, then it shall be reported to the NAA , the State of Registry and the TC holder of the aircraft/part within 72 hours of the person or organisation noticing a reportable condition issue. In this context, serious defects or malfunctions shall mean deviations described in Annexes I and II of EU Regulation 376/2014. At its discretion, the organisation may also report other risks that endanger flight safety or observations. About the announcement are taken copies feedback tracking for. Feedback the observer of the deviation is informed.

### Volunteer reportage To the aviation authority

The organization also commits to the EU Regulation 376/2014 voluntary reporting of safety observations . In the analysis of defects and maintenance observations (2.18), also bears the impact of the defect detection on the actual or possible at risk for the safety of aviation operations . Risk detection and If necessary, the analysis also covers the entire maintenance operation. The aforementioned safety review is carried out at the final maintenance meeting. and recorded in the maintenance records.

Security risk observations that not reported in accordance with a serious procedure, collected and processed in a documented manner at monthly maintenance meetings, where further action and notification to the NAA are decided . Safety notice will be done EASA or NAA by publishing the form.

In all in announcements are at least following information:

- organization name and approval number,

- aircraft and/or component identification information,

- notification target essential maintenance monitoring information,

- made observations and analysis results,

- if necessary defect detection clarify additional information

### Notifications for the operator

Copies of defect reports submitted to the aviation authority (etc.) are provided to the aircraft owner, operator or CAM organization. Any defect findings that are significant in terms of cost are reported to the operator before repair. In addition, the operator receives information about all defects detected at the time of handover for maintenance. and in writing in maintenance documents, where appears also their correction (or move).

### Responsibilities

About the internal reporting system and procedures and training of personnel The technical director is responsible .

## DEFECTIVE AIRCRAFT PARTS RESTORATION IN STOCK

Removed from aircraft for defective some are made usage classification (2.3) and thereto will be attached to it a corresponding accompanying card that follows it until it is repaired or scrapped. After the maintenance is recorded, the part is transferred its status according to to the warehouse follow-up measures for (for repair shipping or scrapping).

## DEFECTIVE PARTS RESTORATION FOR EXTERNAL PARTIES FOR CONTRACT PARTNERS

After maintenance, defective parts are returned to external contractors with an appropriate accompanying card indicating the use classification in accordance with section 2.3.1. Parts that cannot be repaired are mutilated in accordance with section 2.3.5.

## ELECTRICAL MAINTENANCE RECORDING CONTROL

Maintenance records are stored in secure databases in the company's cloud service. The records are continuously backed up and deleted files can be restored for 90 days. Access to the databases is managed by user rights granted by the quality manager. He also monitors that electric accounting is protected from damage, theft and from counterfeiting.

## LABOR RESOURCES AND MAINTENANCE PLAN COMPARISON

Maintenance organization from customers received and otherwise anticipated annual Based on the maintenance order information, the technical director prepares a maintenance plan, for the implementation of which sufficient labor resources are reserved (2.28). The work according to the maintenance plan and the availability of maintenance personnel are presented in an annual calendar-based maintenance and resource plan. The plan is specified according to the customers' advance work orders.

Operational planning of labor resources is carried out for current maintenance work and other tasks . by. Labor resources (+25) %) reserved manufacturers published or/and based on empirical data. In operational planning taken into account also maintenance activities support work (such as administration, training, maintenance and customer service tasks).

Proactive maintenance and resource planning and comparison ensure that the organization has sufficient personnel available to perform the intended work. Personnel utilization planning also takes into account the need for support personnel. The goal is that maintenance work can be make permanent with labor. Possible overload situations being treated work by transfer, overtime or using temporary workers. Overload cases processed in quality meetings.

Maintenance- and resource plans updated possibilities by 3 for a month in advance and the implementation of plans is recorded.

About the procedure answer technical manager.

## CRITICAL MAINTENANCE TASKS CONTROL

### Recurring error critical task

Critical task is born, when the same made by a person repeated welfare is aimed at or affect one to many to the system and A repeated error in the work in question can endanger flight safety.

The creation of critical tasks is prevented through maintenance planning and appropriate task allocation. (i.e. that in question work will be done different by persons). If the above-mentioned it is not possible, so the task performance in question will be subject to an additional check when all are repeated the work stages have been completed.

An additional check point is added to maintenance lists/cards or work and inspection lists if anticipated or it is stated, that critical task situation is born. Likewise will be done, if similar

tasks to perform there is in hand only one person. Acknowledgement point recorded with a stamp or with additional marking (RII / Insp 1 / Insp 2). (RII = Required Inspection Item )

The above. in addition organization there is defined following maintenance situations for good critical tasks, which always undergo the above-mentioned additional inspection:

- If welfare there is been interrupted and in question at the service point there is made between others work by other employees.

- If maintenance site demolition and installation will be done different persons through the agency of, and those involved cannot exchange information at the service point.

Flight safety in terms of critical maintenance tasks (Critical maintenance tasks ):

These maintenance tasks are aimed at systems where an error or malfunction could lead to a safety hazard. Such maintenance tasks are considered critical tasks for which the task upon completion must be done additional inspection by an independent inspector. The additional inspection point is recorded with a stamp or additional marking (RII / Insp 1 / Insp 2) on the maintenance lists/cards in connection with work planning or whenever the task situation in question is identified.

Critical maintenance tasks are:

- Work on control systems, such as installation, assembly and adjustments when parts have been removed/installed from the above systems.

- Engine and propeller installation mixed of the components in question to the activity affect work such as removing/installing parts or calibrating/adjusting.

In addition to the above, maintenance tasks that have been identified as critical in maintenance error analyses or similar observations, such as in the aviation authority's Occurrence Reporting System output, the type approval holder's maintenance bulletins, accident reports, audit observations, operating experiences and training feedback, can be designated for the procedure in question.

Sovereign inspection there is additional inspection, where ensure:

  - detached assemblies are properly installed

  - system works freely size in its area of use

  - of wires right tension

  - control system right activities, e.g. business directions

  - control systems right cooperation size in its area of use

  - software version and fit to that to the airplane

The independent review is carried out after the HTA that approved the critical work has acknowledged the work as completed.

The independent inspector may be the HTA of the machine type or a qualified person approved for this task by the technical director. In exceptional cases, if a qualified independent inspector is not available or not reasonably obtainable, the HTA who performed and signed off on the work may perform the additional inspection in question. ( re-inspection ). Additional inspection will be done however always as a separate work step.

### Area inspection

After the work is completed, the area inspector ensures that no critical work steps have been left unmarked/undone/unacknowledged. and that they have been made properly. If errors are found or deficiencies, they are corrected and reported as a quality observation in accordance with 2.25.

Regarding the above procedures in its entirety, The technical director is responsible for training and use in maintenance work .

## SPECIAL MAINTENANCE AND PRODUCTION METHODS

### Substance unbreakable inspections

The maintenance organization may perform color contrast-based color penetration inspections on aircraft within its scope of work , borescope inspections and composite structures knock checks. The checks are carried out in accordance with the manufacturer's instructions and/or the work instructions drawn up by the organisation. The check may be carried out or supervised by a person qualified for the task (3.11).

The use of the method is limited to work in accordance with maintenance instructions, maintenance bulletins and airworthiness directives, however, the maintenance organization does not perform non-destructive tests that require the person performing them to be qualified in accordance with the EASA- approved standard (Part-145.A.30f). If necessary, the above work is procured from approved suppliers.

### Weighings

The weighing is carried out in accordance with the TC holder's instructions and/or the work instructions prepared by the maintenance organisation. The weighing results are recorded in the weighing report prepared by the maintenance organisation. The HTA is responsible for the weighing task (including filling in the documents).

### Trial runs

The test run 	is carried out 	in accordance with the instructions/requirements drawn up by the aircraft 	manufacturer 	and/or 	maintenance organisation . The test run results are recorded in the test run lists.

Airplane trial run run organization internal trial operation qualification (3.11) a qualified type engineer or a pilot nominated by the operator together with a mechanic.

## MAINTENANCE ERRORS OBSERVATION AND REPAIR METHODS

According to the organization's quality policy, every maintenance person is obliged to report what they have done. or a maintenance error they have detected. A quality report is prepared on the error and the technical director decides on further action.

In order to detect maintenance errors, periodic maintenance includes area inspections, which are carried out on the object during the completion of work in the maintenance area in question. Area inspections are recorded in the maintenance records.

The aircraft's area inspection items include the fuselage, landing gear, engine 1, engine 2, propeller 1, propeller 2, cockpit, cabin, controls, and accessories.

If the area inspector discovers a maintenance error that may cause a defect or malfunction that endangers the safety of the aircraft, in addition to making the repair, he or she will prepare a quality report. The same applies if a similar maintenance error is discovered in another context (such as when performing duplicate inspections (3.7), inspections of critical work (2.23) or inspections of the release of equipment (2.13.4 and 2.16.1). All of the above observations are immediately reported to the technical manager, who is responsible for immediate corrective measures (including prevention of recurrence).

The above-mentioned quality reports are also discussed at the quality meeting, where The impact of each observation on the operation and the necessary follow-up measures are assessed. The assessment takes into account previous experiences maintenance errors, prepared quality reports and deviation reports with feedback. Evaluation results used as observational data quality and human authors -trainings. About the procedure The quality manager is responsible.

## OF THE TURN/TASK EXCHANGE METHODS

In the organization No make shift work.

When exchanging tasks, it is personally ensured that information relevant to the continuity or completion of maintenance is exchanged. at the service point. Relevant information are, among others , what is being done and why, what is the phase of the work and what has been the working method, indirect aspects of what is planned to be done, availability of spare parts and special tools and schedule.

Tasks interruption or exchange recorded maintenance list or work- and checklist. Provided work the successor does not Oh consult work the initiator with at the service location, then the service location will be made additional inspection according to section 2.23 Control of critical tasks.

## SERVICE INFORMATION ABOUT ERROR OR INACCURACIES NOTIFICATION

If an error, inaccuracy or ambiguity is detected in the maintenance data, it is treated as a safety observation and a quality report is prepared, which is reported to the quality manager. At the same time, it is ensured that the error (or equivalent) detected in the maintenance data, along with the preventive measures, is brought to the attention of the organization's personnel. Observations processed and evaluated at the quality meeting (3.1.2.d) and the verified maintenance data error (or equivalent) with background information is reported to the TC/STC holder. Feedback is requested on the above notification, which, upon receipt, is presented to the person who observed the error (or equivalent). The technical director is responsible for the procedure.

## PRODUCTION DESIGN METHODS

Production planning is based on annual aircraft maintenance plans received from contract customers, which are based on the aircraft's operating and flight plans. Work needs of other customers is anticipated based on experience. The above. information by are assembled maintenance organization maintenance plan, whom for implementation reserved sufficient labor resources. Provided work orders If more than expected comes in, resources will be increased accordingly.

### Maintenance activities and human resources use are planned next by procedures:

- Annually:

  - is being prepared maintenance plan predictable maintenance work scope and of times by,

  - ensure personnel adequacy and operating principles on an annual basis,

  - are being checked maintenance capabilities adequacy and for in step with feeling,

  - maintenance requirements, -responsibilities or -contracts changing updated plans.

- Monthly:

  - updated maintenance plans next for three for a month,

  - updated work orders required maintenance capabilities,

  - ensure resources adequacy predictable work orders to perform,

  - are planned resources other use (incl. quality- and training plans obligations),

  - are being checked quality deviations corrective measures progress,

  - processed new quality reports with follow-up measures.

- Weekly:

  - is decided urgently tasks for completion,

  - is decided individual maintenance and others tasks order of execution in hand existing resources within the framework considering humaneness of personnel performance trammels.

Maintenance- and a human resource plan is made and maintained under According to 2.22. The technical director is responsible for production planning.

# PART L2: LINE MAINTENANCE CONCERNING ADDITIONAL METHODS

Line maintenance platform:

The line maintenance stations approved for use by the organization (1.8.2 and 5.3) are manned for the duration of the line maintenance work. The requirements for maintenance operations are the same as for workshop maintenance. Permanent line maintenance stations are equipped with standard tools and supplies as well as general maintenance equipment (including work platforms and spotlights). Temporary line maintenance stations are equipped with the above-mentioned equipment for the duration of the operation. Special tools and maintenance literature are brought from the workshop to the extent required for maintenance.

## L2.1 AIRCRAFT PARTS, EQUIPMENT, TOOLS, EQUIPMENT ETC. CONTROL

The tools and equipment at the line maintenance location are listed in the workshop's accounting records with their location. The quantity and condition of the equipment and tools are checked during maintenance using the maintenance location-specific accounting list. Missing supplies are replenished and defective tools are replaced without delay. Tools and supplies are stored in locked rooms or cabinets. Parts and equipment are not stored at the line maintenance location (relying on the workshop's warehouse). Replacement parts and equipment are packaged and stored appropriately and the related inventory records are updated after returning to the workshop.

## L2.2 REFUELLING, FOR DEICE ETC. RELATED METHODS IN LINE MAINTENANCE

Refueling and deicers No make (excl. maintenance for implementation related refueling, which will be done with the aircraft operator's quality criteria and methods).

## L2.3 DEFECTS AND REPEATED DEFECTS TRACKING IN LINE MAINTENANCE

Monitoring of faults and recurring faults is carried out according to workshop procedures (separate records are not kept at the line maintenance site for these matters). Maintenance documents with fault information are also handed over to the operator according to workshop procedures. This monitoring is carried out, if necessary, on the basis of all maintenance and operating observations.

## L2.4 TECHNICAL TRAVEL DIARY FILLING METHOD IN LINE MAINTENANCE

Let's do it workshop methods by.

## L2.5 POOL AGREEMENTS AND LISTED PARTS CONCERNING METHODS

No touch.

## L2.6 FROM THE AIRCRAFT DISCONNECTED DEFECTIVE PARTS/EQUIPMENT CONTROL

Defective assemblies and equipment are equipped workshop methods (MOE 2.3.1) in accordance with with tags (with escort cards) immediately after removal from the aircraft. At the same time, other necessary documents are prepared and properly packed for transport to the repair shop. After returning to the repair shop, the parts and equipment in question are taken to storage for further processing.

## L2.7 CRITICAL TASKS CONTROL AND ERRORS REPAIR METHODS

Let's do it workshop methods by (see paragraph 2.23 and 2.25).

# PART 3: QUALITY METHODS

## QUALITY SYSTEM

#### Quality assurance

The framework of quality assurance is comprised of recurring audits and quality checks, as well as the corrective, preventive and development measures they produce. In addition, every person in the organization is required to report observed quality problem. Quality management system second as part of there is management monitoring and feedback system (see 3.1.4 for more details).

Auditing refers to a procedure whose purpose is to independently monitor compliance with standards and requirements and the adequacy of methods to ensure that the methods used: promote good maintenance practices and maintain aircraft and its equipment/parts airworthiness . The independent audit should provide an objective description of the operations of the entire organization.

With quality control (with spot audits) means defined task, maintenance work or examination of a document, etc., to determine that appropriate requirements are followed when performing a task and that the required quality level or its prerequisites are achieved. The acceptance requirement for the inspection is given in the maintenance instructions or in manuals or is determined on a case-by-case basis.

Quality observation there is organization each personnel continuous to work, to the activity, to the equipment and critical observation of conditions to identify and address a quality problem or unsatisfactory performance.

#### Schedules

- The planned execution times for audits (methods, product lines and line maintenance) are presented in the quality plan. The planning period is one calendar year. The plan was drawn up by the quality manager and is approved at the management review meeting. If the plan cannot be followed, the matter is discussed at the monthly maintenance meeting, where a replacement time or procedure is agreed. The quality manager is responsible for updating the plan and the agreed actions. If the audit period for a sub-area is exceeded, so organization border respectively in question area scope of work, thereto until when an approved audit has been completed.

- Quality checks will be done own annual program by mixed if necessary. Inspections is carried out by the quality manager or a person designated by him. Each inspection is documented and any deviations detected are analyzed and reported.

If an actual or potential risk is identified during the analysis of inspection findings or quality deviations aviation safety, so the matter processed in a documented manner quality meeting. The meeting decides on the follow-up action to the safety observation or occurrence and on reporting to the NAA . Reporting is done in accordance with the voluntary reporting of EU Regulation 376/2014. The report is made using a form published by EASA or NAA .

- Maintenance monthly meeting (or quality meeting) processed all current affairs quality issues such as the implementation of the quality plan, quality and safety observations, and quality and deviation reports with follow-up measures. The meeting also discusses feedback, reports and follow-up measures on defects that endanger flight safety and hinder airworthiness (2.18), maintenance errors (2.25) and maintenance data errors (2.27), as well as other safety issues that arise.

The monthly maintenance meeting (or quality meeting) is organized when necessary, but at least every 1 - 2 months. The quality manager is responsible for organizing the meeting. The quality meetings are attended by the responsible persons of the organizations and persons separately invited by them. The meeting can also be organized in cooperation with the company's Part -CAMO organization.

- Management judgment will be done calendar year and evaluation meeting is held annually in January (or in addition if necessary). At the evaluation meeting is accepted also new annual plans.

#### Reporting procedures

### Quality report:

A quality report is prepared for a quality inspection, quality observation, safety risk observation, customer feedback, change proposal or similar. The quality manager is responsible for processing the report, monitoring the measures and compiling statistics. The technical director is responsible for the corrective actions required by the reports. Quality problems are handled in accordance with the principles of 3.3.2 deviation analysis.

### Auditing reportage:

Audit results recorded to the audit protocol. Quality deviation is being prepared A deviation report is prepared from the audit, to which is attached audit report and deviation reports.

### Reporting forms:

Quality and auditing activities are reported using forms or in a free format. Standard forms include a quality report, a deviation report, a method audit report and a product audit report, samples of which are in the form folder. Practical reporting can be done either electronically or in written form.

### Management evaluation report:

Management evaluation is being prepared separate evaluation report. About the procedure answer responsible manager.

### Quality documents conservation:

Documents appropriate supervision, recording and archiving answer quality manager.

#### Management tracking- and feedback system

Next by procedures ensure, that responsible manager weather sufficiently information quality obligations and -activity. Responsible manager answer last on, that detected quality problems are being investigated and repaired within a reasonable time in time and unsatisfactory developments are addressed and The quality of operations is systematically improved.

### Practical activities

The progress of corrective actions, the completion of quality tasks and compliance with deadlines are monitored by the quality manager. To support the monitoring, he maintains a monitoring list, which presents all quality tasks to be monitored and their deadlines. The quality manager is obliged to expedite or demand the above-mentioned measures, if they threaten to be delayed reconciliation. He there is also obliged to keep the organization's leaders over time up to date with the monitoring situation of the above matters mixed current quality events (including quality inspections). Quality tasks and deadlines are reviewed at a quality meeting attended by the organization's responsible persons.

### Audits and correct measures

The responsible manager participates in audit meetings (discussion of audit results) and approves the auditor's audit report with his/her signature. He/she also participates in monthly meetings to review e.g. quality deviations corrective working progress and deadlines compliance.

### Management judgment

The management assessment is carried out annually under the leadership of the responsible manager. The quality manager is responsible for preparing the assessment and the responsible manager for reporting. The procedure compiles and evaluates the results of the maintenance organization's quality activities and other similar information and observations, and based on the conclusions drawn, decisions are made on general revisions of operations, methods and development policies, as well as new goals. The review of the results of quality activities includes a quality deviation and joint analysis, which aims to identify and clarify the root causes of possible wider quality problems.

## METHODS AND AIRCRAFT QUALITY AUDITS

Each area of maintenance operations is audited every calendar year. The annual audit is conducted as complementary method and product line audits. Locations are audited separately. If necessary, a change audit is conducted.

In method auditing let's make sure thence, that to the manual approved procedures are

- sufficient and unambiguous,

- fulfill set standard and

- them are followed practically.

The product line audit checks that the maintenance capabilities (MOE 1.9), products, and maintenance process are requirements in accordance with mixed maintenance certificates there is given duly. Product lines updated as operations change. Product lines include:

- Machinery under 2730 kg,

- 2- engine maintenance,

- Equipment maintenance (point 1.9.2 in extent).

Procedures with the same content for different product lines can be audited on a random basis. If a deviation occurs , the corresponding procedure must be audited for each product line. The product line audit also includes monitoring the ongoing practical maintenance work under real conditions (e.g. day/night) in accordance with the organization's policy.

Equipment maintenance product line audits is aimed at to do in question product line maintenance work in connection with. If no suitable maintenance work is known, the audit is carried out based on maintenance readiness and maintenance documents.

## AUDITS AND DEVIATIONS REPAIRING MEASURES

#### Auditing

Auditing in preparation let's get acquainted:

      - quality plan for implementation and quality system functionality,

      - quality meetings and management review minutes,

      - previous auditing quality deviations for processing and corrective measures,

      - quality reports (observations and inspections) observations and corrective working results.

If necessary, the above-mentioned reviews will be deepened. and expanded through interviews with responsible persons. In addition, the auditor checks that the audit protocol is in accordance with the maintenance organization's scope of work and the latest Part-145 requirements.

### Audit inspection

The auditor conducts the audit in accordance with generally accepted auditing principles. He or she may use checklists, interviews, and monitoring of practical activities.

Audit protocol recorded requirement-specific ratings:

      - manual descriptions (and possible (with additional instructions) sufficiency and clarity,

      - norms fulfillment (Part-145 standard and their official interpretations),

      - practical activity of correspondence procedural descriptions and to standard requirements mixed that The activity in question takes place under the leadership and supervision of a designated responsible person.

The auditor can make additional audits outside the audit program on a random basis and/or based on non-conformance observations. All relevant observations are recorded in the observation column of the audit report and potential quality deviations are identified and discussed at the audit meeting for.

### Audit meeting

The audit meeting is attended by the organization's responsible persons and the auditor. The quality manager is responsible for arranging the meeting, and the auditor presents the audit results. The aim is to examine the observations recorded as quality deviations in detail. The meeting also takes a position on the comprehensiveness and adequacy of the audit performed and the need for additional audits.

### Audit report

Based on the audit results and the meeting discussion, the auditor prepares an audit report in which he lists the quality deviations. classified audit findings. To each quality deviation recorded concerning it handbook- and requirements mixed deviation quality class and recommendation corrective schedule of actions according to the table below. The deviation description is prepared in such a way that it enables corrective actions working definition and starting. To the audit report is attached audit protocol, which presents audited destinations and made quality observations. Auditor deliver audit report with attachments to the maintenance organization within two weeks of the audit.

| Deviation quality class is defined next on the grounds: |  |
| --- | --- |
| Serious deviation | Terms and conditions/regulations not complying to leave or flight/occupational safety related perception, which require immediate or short time period corrective measures. |
| Mild deviation | As above, but require measures in reasonable on schedule (up to 3 kk), but the deviation does not pose any safety risks and is not contrary to the operating conditions. |
| Comment | Previous milder perception, which require subject matter inspection, clarification or re-evaluation by the next annual audit. |
| Repeated deviation | Reference information from the past similar from the observation of an anomaly, whereof there is reported last the two of you year during. |

#### Corrective measures

Once the audit report has reached the maintenance organization, it is reviewed by the technical director and the quality manager, who is responsible for presenting the report and the review results to the responsible manager. Based on the above-mentioned processing, the quality manager prepares the deviation reports required for the management of corrective actions for each deviation and determines their further processing.

If the above-mentioned reports or their review identify the need for urgent action, the responsible manager will decide on further action in cooperation with the technical manager. The necessary deviation analyses and corrective actions will be carried out. without delay the procedure below in accordance with.

Deviation reports are handed over to the Technical Director for deviation analyses. Each analysis aims to identify and isolate the direct and contributing factors that contributed to the occurrence of the deviation and their root causes. Corrective actions are targeted to the cause of the issue. in order escort alongside also deviation background factors and to the root causes. The goal is to prevent the recurrence of the deviation. A deviation analysis and corrective action plans are made within two weeks of receipt of the audit report. The results of the analysis and corrective actions, including schedules, are recorded in the deviation report.

Corrective activities will be done plan by technical director under maximum 3 month within. The quality manager supervises the handling of the deviation and the implementation of corrective actions. If they are delayed, he is obliged to demand or expedite them. If, for some justified reason, the corrective actions cannot be completed by the deadline, the responsible manager may grant an extension. A quality report is prepared on the matter, which records the transfer basis, the new deadline and the approval of the responsible manager, and the report is attached to the deviation report in question.

Once the corrective actions are completed, the Technical Director approves their effectiveness and adequacy with his/her signature. The Quality Manager then confirms the adequacy and appropriateness of the corrective actions. performance and accepts they with his signature, which also decide (closes) report. Report can be decide (close), if measure while staying its tracking transferred to the rest of the organization to the plan, which there is quality manager or responsible leader under the supervision of. Supervision

The transfer is recorded in the report. If the effectiveness and adequacy of the corrective actions cannot be reliably assessed, it can be transferred to a follow-up audit. The transfer, its supervision and implementation time are decided by the quality manager.

The quality manager maintains a monitoring list of all quality tasks to be monitored and presents their status. if necessary (quality meetings, management information, audits, official inspections). If If corrective actions for a deviation are not taken or are left incomplete or delayed unreasonably, the quality manager transfers responsibility for monitoring the deviation to the responsible manager. The transfer is made in writing with a quality report.

#### NAA announced observations handling

Aviation Administration in writing announced observations processed direct their having arrived under the leadership of the director. The discussion shall state the subject and urgency of the corrective actions ( ref . 145.A.95 a and b). The discussion shall also decide on immediate actions to prevent the possible adverse effects of the deviation before actual women corrective working graduation. Corrective actions The decision is made by the technical director or, if necessary, the responsible director. Analysis of observed deviations and corrective actions are carried out in accordance with the principles of section 3.3.2. The actions are reported according to the deadlines and conditions announced by NAA .

Maintenance organization take into account, that level 1 regarding observation NAA is obliged to take immediate measures to revoke or restrict the organisation's operating licence (temporarily, partially or completely) until the organisation has successfully completed the necessary corrective measures.

## QUALIFICATION AND TRAINING METHODS FOR CERTIFYERS AND SUPPORT PERSONNEL

#### Qualification methods

The person issuing the certificate of maintenance and the support personnel must be at least 21 years of age and have the following qualifications for their HTA role : authorization, authorization The prerequisite for expansion or renewal is:

      - authorization according to aircraft and/or equipment mixed their maintenance sufficient knowledge,

      - maintenance organization activity and methods knowledge,

      - suitable maintenance experience at least 6 month last the two of you year during,

      - completed Human Factors training ( ref . 3.13),

      - sufficient recurrent training the two of you last year during ( ref . 3.4.2),

      - estimated competence, eligibility, ability and language skills make their duties in accordance with work (verification of the matter may include monitoring of maintenance work and demonstration tests),

      - B1/B2/B3/C -persons HTA authorizations required qualification In their AML license.

Job qualifications (Class A functions)

      - maintenance organization Oh also grant for non-type trained B1 and B3 class For HTA and for category A and B2 maintenance personnel, HTA authorisations in accordance with their licence category subgroups for planned line maintenance tasks or simple defect repairs within the scope of AMC.A.145.30(g). Task authorisations are granted based on specific training , work experience and if necessary to be held screen test by mixed for category A and B2 persons, also compliance with the requirements of points 145.A.35n+o.

Equipment maintenance HTA authorization as a prerequisite there is, that authorized there is industry in addition to basic education in question maintenance work required additional training mixed sufficient work experience and recognized In addition, the person to be authorized must be subject to recurrent training in accordance with the requirements (3.4.2).

One-time HTA authorization can be grant

- to an employee who has a corresponding HTA rating for an aircraft of similar technology, structure and systems , or

- to a person with at least 5 years of work experience in maintenance work and a valid ICAO aircraft maintenance personnel licence and type rating for the aircraft in question, if applicable the establishment does not have a person capable of the task in question, who has a license and approved maintenance organisation. The maintenance organisation shall obtain and maintain evidence of the experience and licence of the person concerned.

One-time HTA authorization admission will be announced For NAA 7 day within.

A limited HTA authorisation may be issued in connection with a PFI inspection for an approved inspection that: require maintenance certificate (MTOE 2.16.1). Authorization can be grant aircraft to a commander or flight engineer based on specific training and a performance test, if the technical director is satisfied, that competent person remains to perform airworthiness directive as required by the procedure. Authorization is granted for a maximum of one for a year.

### Delegation

A review document is prepared for the HTA authorization, stating that the requirements have been met. The authorization expansion and renewal in connection with will be done equivalent review. Person HTA authorisations, with their scope and limitation definitions, are granted by means of an authorisation certificate issued by the maintenance organisation, which may also include other specialist job and task qualifications. In addition, the HTA list (5.5) is updated as necessary.

Maintenance certificate givers qualification review and authorization answer quality manager. HTA receives a copy of the authorization certificate with the corresponding information. He is obliged to present the authorization to the competent aviation authority within 24 hours upon request.

Authorizations and qualifications are fixed-term and in effect maximum maintenance mechanic until the expiry date of the licence (excluding one-time and limited HTA authorisation).

#### Training methods

Induction training: A new employee is introduced to the organization's operations and duties during induction training, whom content defines technical manager. He takes care of also, that for the new task transferee weather necessary induction training. The induction is recorded in the training documents.

Refresher training is training related to the management of maintenance capabilities and changes in the scope of work (e.g. type, maintenance method and special equipment courses), which is arranged, if necessary, taking into account the requirements set for the training in question by the aviation authority.

Recurrent training is organized according to personal study needs and plans. The technical director is responsible for the training. Training achievements are recorded in the person's training documents. The recurrent training areas are:

- Technical data: Changes to type-specific maintenance procedures and technical data/work instructions (incl. Fuel Tank Safety/CDCCL and EWIS instructions/methods, if they apply to the type of machine in question).

- Methodology training: Part-145 requirements and MOE methods (repeat and changes).

- Safety training: Human factors, limitations of human performance and the maintenance organization's deviation, error, quality and occupational safety observations with corrective actions .

Each in the educational area will be organized education at least 8 B the two of you year during or at least 4 hours annually. The additional training needs of permanent personnel are assessed in the annual training planning and, if necessary, the scope of refresher training for part-time personnel is decided by the technical director on a case-by-case basis at the beginning of the work period. The implementation and adequacy of the refresher training are reviewed in the annual audit and management review.

Recurrent training is organized interactively so that feedback from the training is recorded and processed by the quality organization as corrective measures and information to support learning and development.

### Training plan

To be arranged designed by trainings are assembled education to the annual plan, whom The technical director is responsible for implementation and supervision.

### Education organizing principles

A course in line with the educational objectives can be compiled from various educational events, such as training courses, educational events, information sessions, seminars, if their content corresponds to the educational objectives and the teaching is valid. The training is organized as an in-house activity or acquired from a reliable source. from an educational organization. Education can be arrange also derived Self-study. Self-study is limited to rarely used working methods or exceptional situations if a qualified teacher is not available through reasonable arrangements.

The content, teacher(s), participants, time and duration of the training provided are recorded, as well as an assessment of the achievement of the training objectives. If an exam is given for the training, the results are recorded. In addition every participant ensure their participation for education with one's own hand with a signature on the student list for the training event. The training achievements are combined and recorded in the training records.

### Responsibilities

Educational planning, -arrangements, -supervision and -accounting answer technical manager.

Technical manager there is obliged to follow its staff professionalism and If necessary, arrange proficiency tests and issue orders for additional training.

#### Type training internship organization

The maintenance organisation may, within its scope of work, arrange practical training for type training in a manner approved by the NAA on a case-by-case basis. Training activities are currently limited to persons applying for an additional rating in a category/subcategory belonging to aircraft group 1 (66.A.45). Training includes work exercises and assessments ( practical + assessment ).

The supervisor and assessor of the practical training tasks are required to be a certifying officer for the relevant aircraft type. They must also be familiar with the Part-66 requirements for practical training and practical examinations, as applicable. ( assessments ) for completion given standard (more specifically Part /AMC/GM-66.A.45 and the instructions for organizing training and assessments mentioned below).

Practical exercises experience ) is organized in accordance with Part-66 Appendix III, section 3.2. Exercises is chosen so, that they comprehensive at least 50 % in question point The tasks shall provide a comprehensive cross-section of maintenance tasks essential to the type of aircraft and shall also include demanding maintenance tasks. Their selection shall take into account subjects that cannot be adequately taught in theoretical training.

The assessments will be organised in accordance with the requirements of Part-66 Appendix III, Section 4.2. The assessments will be carried out by named evaluators, with which there is proper competence. In the evaluation are being reviewed test knowledge and skills. In addition are taken into account instructions that​ are given in Part-66 Appedix III ” Aircraft Type Training and Examination Standard ” and AMC-66 Appendix III ” Evaluation of the competence: assessment and assessors ”.

The work exercises and assessments, including their approvals, are recorded in a logbook , which has been prepared based on the Part-66 Appendix III, section 3.2 task list and specified according to the essential maintenance tasks of the aircraft type in question. The logbook template is presented in the attachment.

About the internship procedure answer technical manager. He names also work exercises instructors and assessors, whose names are recorded in the logbook . The trainee is responsible for applying for a type rating from the NAA . A copy of the logbook is taken, which are recorded for the organization's training records.

## PERSONAL REGISTER

Maintenance organization maintain from its personnel following information:

- name, date of birth and ID information mixed job description or task

- personal identification number (for security reasons) separately corporate in the personal register)

- aviation industry work experience (+copies of certificates),

- basic and vocational training (+copies of certificates),

- type- and special work training (+copies of certificates),

- recurrent training,

- authorizations and qualifications,

- copy in effect from the existing maintenance mechanic from the license.

The above information (excluding personal identification number) is presented in personal training cards (1 and 2), copies of certificates and licenses, and authorization and qualification documents. Aviation authorities, organization responsible persons and auditors have the following information related to their duties: the right to inspect register information.

The above. information maintain quality manager and them preserved locked in the state. When to the register is attached new qualification documents, copies of the certificates are taken from the original by the quality manager. He is also responsible for checking the accuracy of the documents.

Employees have the opportunity to access and obtain a copy of their personal information upon request. The documents in question are kept for three years after the person has left the organization .

## QUALITY ASSESSORS

Internal quality audits of the organization are carried out by an independent auditor (named in section 1.3.3). Quality audits (spot checks) does quality manager or his named expert. Responsible persons can also conduct quality checks related to their duties.

## INSPECTORS QUALIFICATION

Service certificate issuer job description included following maintenance operations inspection tasks:

- double check (unqualified) person to do and to acknowledge work countersignature).

- regional inspection (aircraft) point 2.25 by.

- critical work inspection point 2.23 by.

- maintenance certificate administration check (only named HTA).

In addition HTAs work if necessary class B1 or B2 as a support person, whom task there is where necessary, ensure that all necessary tasks or inspections have been completed in accordance with the requirements before the category C authorised maintenance staff issues a certificate of release to service.

## PERSONNEL QUALIFICATION

Maintenance certificate providers and qualification of support personnel and authorization is made According to 3.4.1.

- class B1 maintenance certificate issuers work also as a foreman and as a work planner.

- class B1 and B2 maintenance certificate issuers work if necessary as a support person.

### Mechanic

A person with appropriate professional training and/or sufficient practical experience can be qualified as an installer. Familiarity with maintenance work. For installer can be accept also individual work or a specialist in that area, when he or she has recognized specialist skills for the work or task in question. The installer qualification is entered in the personnel register (training card 1).

The installer's areas of responsibility and qualifications are defined by the technical director, who can also grant the installer clearance qualifications for maintenance tasks. Qualification requires that the installer has completed technical director to define task training and screen test (if necessary).

Part-147 student or other intern Oh run in the organization practical internship as part of their studies, when this has been agreed upon in the internship agreement. The intern is required to have knowledge of the basics of maintenance operations and basic skills as given in the studies. The technical director decides on the assessment of their adequacy and the scope of the orientation training on a case-by-case basis. The intern always carries out maintenance internship under the supervision of a supervisor appointed by the technical director.

### Qualifications and authorizations control:

The validity of licenses and ratings/authorizations is monitored electronically and their expiration information is notified to the person in question in advance. The renewed license must be submitted to the quality manager, who updates the monitoring information and takes a copy of it to the company's file and renews the ratings/authorizations. The quality manager is responsible for monitoring.

## AIRCRAFT AND DEVICES MAINTENANCE EXCEPTIONAL PROCEDURES POSSESSION

Maintenance of aircraft, equipment and tools shall be carried out in accordance with the maintenance programme of the aircraft in question or the maintenance instructions published by the type approval holder. If there is a need to deviate from the approved programme or instructions, this may only be done with the permission of the aviation authority or the type approval holder/manufacturer. The change shall be applied for by the aircraft operator. operator.

Machine type HTA Oh if necessary draw change needs analysis, whereof appears:

- why approved instructions or maintenance programs let's digress,

- how will come to act work doing, inspection and supervision regarding,

- negotiations and agreements operator, manufacturer and aviation authority with.

The Technical Director ensures that the exception procedure has appropriate approval before it is implemented. Incident reports are recorded in the aircraft's documentation.

## ORGANIZATION ABOUT PROCEDURES DEVIATION POSSESSION

Maintenance operations follow the instructions and methods in this manual (MOE). If there is a need to deviate from the work and operating methods in this manual, the approved responsible person, after consulting the quality manager, will prepare an exception application stating:

- why this manual instructions and methods are deviated from,

- how will come to act procedures, inspection and supervision regarding,

- what there is negotiated and agreed NAA with.

Exception procedure is accepted At NAA and its commissioning measures corresponding Technical Director. The exception report is attached to the organization's accounting records.

## SPECIAL QUALIFICATIONS

Organization Oh grant following special work qualifications case-by-case based on the qualification review :

- Substance unbroken inspections qualifications (limited 2.24.1 according to) :

a) Penetrant dye inspection , b) borescope inspection , and c) laminate structures Knocking check. Qualification as a prerequisite there is the tools of the inspection method in question and knowledge of instructions mixed practical skill, whom technical manager Oh if necessary ensure by display test.

### Mechanic trial operation qualification

Maintenance person trial operation qualification granted by machine type.

The qualification requires an HTA authorization for the aircraft type in question, appropriate task training, knowledge of safety regulations and a limited radio operator qualification, as well as practical skills approved by the technical director.

Qualifications grant quality manager after hearing technical leader. Qualification maintenance requires the maintenance of professional skills through work performance and recurrent training. The matter is reviewed annually as part of the person's refresher training planning. If necessary, the maintenance of professional skills can also be ensured by a demonstration test, which is decided by the technical director.

## FOREIGNERS MAINTENANCE WORKING GROUPS CONTROL

Supervision of work groups will be organized case by case by the technical director. The external team must always provide a Part-145 maintenance certificate for the work performed work.

## HUMAN FACTORS - TRAINING METHODS

Basic human factors and human performance training is provided in accordance with the content requirements of GM 145.A.30e, taking into account the methods, procedures and circumstances of the organisation.

The need for basic training is assessed on an individual basis, taking into account the person's previous education and work experience as well as current duties. Supplementary basic training is arranged for current personnel before the entry into force of the relevant regulation and for new personnel always in connection with the orientation training.

Recurrent training will be organized 3.4.2 in accordance with. In educational content emphasis is placed on:

- organization appropriate quality- and safety observations,

- suitable others quality- and safety information/observations,

- GM 145.A.30e contents (maintenance organization activity concerning partly).

## PERSONNEL QUALIFICATIONS JUDGMENT

Person competence in relation to intended tasks are evaluated his training- and based on their experience. The quality manager is responsible for the assessment procedures and their documentation. He/she is responsible for assessing the qualifications of the maintenance personnel. The technical director shall be consulted. The assessment procedure shall utilize the AMC 1 145.A.30e guidelines.

The assessment of competences is carried out when the person joins the organisation and, if necessary, in connection with a change. In this context, induction training is organised (3.4.2). In addition, the competences maintenance the assessment is made annually in connection with training needs planning (3.4.2).

The training and experience information related to a person's duties is compiled on training cards, which present: also intended functions and job description mixed person qualification and recurrent training situation. The certificates related to the above information are attached to the organization's personnel register 3.5 in accordance with.

Based on the training and experience data and the interview, a training plan is drawn up for the person required for the main and ancillary tasks, which is arranged as supplementary and recurrent training (3.4.2). The plan is reviewed and updated annually or as necessary. In addition, the management of tasks is supported if necessary, with specific training and support staff arrangements.

The assessment of suitability for the tasks is carried out in accordance with the criteria mentioned below, the competence assessment utilizes the Competence section of GM 2 145.A.30 (e). assessment procedure list. The authorisation review of maintenance certifying officers (including assessment of competence) is carried out in accordance with 3.4.1 and recorded on the authorisation review form. The assessment of other personnel is recorded on an open form using the criteria by. Job qualifications/authorizations granted 3.4, 3.7, 3.8 and 3.11 according to procedures . The start of a new task always includes orientation and guidance, and the person gradually moves to independent work at the discretion of the technical director.

In addition to the above, the technical director and quality manager continuously monitor the qualifications of the personnel related to their duties and are obliged to take further action if necessary. The monitoring also takes into account factors that support the maintenance of human performance.

### Admitted responsible person:

- technical basic education and aeronautical industry work experience,

- managerial and administrative experience,

- Part-145 and Part -M requirements knowledge,

- maintenance organization maintained equipment and used methods knowledge,

- authority seal of approval in question to the task.

### Maintenance certificate giver and maintenance support person:

- qualifications judgment will be done point 3.4.1 in accordance with.

### Quality Manager and auditor:

- suitable basic education and industry work experience,

- audit/assessment training and - experience,

- Part-145 and Part -M requirements knowledge,

- maintenance organization equipment and methods knowledge,

- on demand regulatory approval in question to the task.

### Installers and other to the activity participate cast:

- suitable basic education and industry knowledge,

- task area knowledge or individual task/job special expertise,

- involvement refresher training tasks demanding in extent,

- on demand regulatory approval in question to the task.

### Special work qualifications

- defined in the 3.11.

### Part-147- student

- defined in the 3.8.

# PART 4: MAINTENANCE AGREEMENTS

## MAINTENANCE CONTRACT OPERATORS

No be.

## OPS OPERATOR REGARDING TO BE FOLLOWED METHODS AND PAPERWORK

To be agreed between the guardian and the operator between in the maintenance contract. The contracts are in a separate contract folder, the technical director is responsible for keeping.

## OPS OPERATOR DOCUMENTS FILLING

To be agreed between the guardian and the operator between in the maintenance contract. The contracts are in a separate contract folder, the technical director is responsible for keeping.

# PART 5: APPENDICES AND LISTS

## APPENDICES AND FORMS

Business premises floor plans mixed form list and its according to master forms are in the form folder maintained by the technical director (presented upon request).

## CATALOG SUBCONTRACTORS

Maintenance organization No use subcontractors.

## CATALOG TEMPORARY LINE MAINTENANCE POINTS

| Service point name and address | Run |
| --- | --- |
| No in use |  |

  - CATALOG SUBCONTRACTING

Maintenance organization No use subcontractors.

  - MAINTENANCE CERTIFICATE GIVING (HTA) CATALOG

| NAME | PERMIT NO | GROUP 3 | BEECH 90 SERIES | CESSNA 525/525A/C525B | BOMBARDIER CL600-2B16 | COMPONENT- WORK |
| --- | --- | --- | --- | --- | --- | --- |
| Anton Anonymous | FI.66.123456 | B1 / B2 / C | B1 / B2 / C | B1 / C | - | C14 |
| Tero Terava | FI.66.234567 | B1 / B2 / C | B1 / C | B1 / B2 / C | - | C14 |

Anonyymi OY

FI.145.9999

MAINTENANCE ORGANISATION EXPOSITION

MOE-LIITELUETTELO

Muutos 17

3.11.2025

Sivu 50

| LOGO | FORMS NAME | MOE REFERENCE | NUMBER OF PAGES | REVISION LEVEL |
| --- | --- | --- | --- | --- |
|  | PART 1 FORMS |  |  |  |
| Traffic LU3812 | EASA FORM 2 (OPERATION LICENSE APPLICATION FOR AMENDMENT) | 1.10 | - | - |
| Traffic LU3813 | EASA FORM 4 (APPLICATION RESPONSIBLE PERSON) | 1.10 | - | - |
| MOE 1.8- 1 | SERVICE SITE REPORT | 1.8.2, 2.13.5 | 1 | April 15, 2021 |
| MOE 1.9- 1 | MAINTENANCE READINESS REVIEW | 1.9.1 | 2 | April 15, 2021 |
|  | PART 2 FORMS |  |  |  |
| MOE 2.13- 1 | CERTIFICATE OF RELEASE TO SERVICE (CRS) | 2.13.1- 4 | 1 | April 15, 2021 |
|  | PART 3 FORMS |  |  |  |
| MOE 3.1- 1 | QUALITY- AND TRAINING PLAN | 3.1.2, 3.4.2 | 1 | April 15, 2021 |
| MOE 3.1- 3 | AUDIT PROTOCOL (METHODS) | 3.2, 3.3.1 | 5 | April 15, 2021 |
| MOE 3.1- 4 | AUDIT PROTOCOL (PRODUCT LINE) | 3.2, 3.3.1 | 3 | April 15, 2021 |
| MOE 3.4- 1 | HTA AUTHORIZATION REVIEW | 3.4.1- 2 | 1 | 1.8.2021 |
| MOE 3.4- 2 | HTA AUTHORIZATION | 3.4.1- 2 | 1 | April 15, 2021 |
| MOE 3.4- 3 | RIGHTS OF SET-OFF | 3.4.1- 2 | 1 | 1.8.2021 |
| MOE 3.5- 1 | TRAINING AND EXPERIENCE (TAIL 1 + CARD 2) | 3.5 | 2 | 1.8.2021 |
|  | PART 4 APPENDICES |  |  |  |
|  | No be |  |  |  |
|  | PART 5 APPENDICES |  |  |  |
| Annex 5.1- 1 | MAIN POINT OF OPERATION VANTA | floor plan | 1 |  |